Synchronous improvement of methane production and digestate dewaterability in sludge anaerobic digestion by nanobubble

Supplementary materials

Fig.S1 Plot of PCA analysis of archaea (a) and bacteria (b)

Fig.S2 Changes in EPS during anaerobic digestion

Fig.S3 3D-EEM maps of EPS of digestate

Table S1 Properties of inoculum and substrate

|  | Seed sludge |  | Dewatered sludge | Initial sludge (mixture of Seed sludge, DWS, and NBW) |
| --- | --- | --- | --- | --- |
| TS (%) |  | 5.4±0.7 | 13.1±1.0 | 6.4±0.5 |
| VS (%) |  | 2.5±0.3 | 7.4±0.5 | 3.5±0.1 |
| VS/TS (%) |  | 46.1±1.3 | 56.7±0.3 | 55.6±0.8 |
| pH |  | 7.64±0.12 | 7.25±0.09 | 7.33±0.02 |
| SCOD (mg L-1) |  | 2803.2±27.8 | 4168.1±60.6 | 2994.9±67.4 |
| TAN (mgN L-1) |  | 1351.0±71.4 | 831.6±40.6 | 477.4±7.0 |
| FAN (mgN L-1) |  | 71.5±0.1 | 18.2±0.1 | 12.7±0.1 |
| SRF (1012 m kg-1) |  | - | - | 6.5±0.9 |
| CST (s) |  | - | - | 15.0±0.4 |
| MC (%) |  | - | - | 85.6±0.2 |
| C (%) |  | 29.6±1.0 | 22.8±1.4 | - |
| N (%) |  | 2.3±0.1 | 3.0±0.1 | - |
| H (%) |  | 4.3±0.1 | 3.7±0.1 | - |
| S (%) |  | 0.61±0.03 | 0.76±0.01 | - |
| C/N |  | 13.0±0.2 | 7.6±0.1 | - |

Note: The initial sludge mass mixing ratio was DWS: NBW: seed sludge = 682g:682g:136g and the initial sludge properties are based on five operating conditions; TS: total solid; VS: volatile solid; SCOD: soluble chemical oxygen demand; TAN: total ammonia nitrogen; FAN: free ammonia nitrogen; SRF: specific resistance to filtration; CST: capillary suction time; MC: moisture content is based on dewatered sludge cake; Determination of C, N, H, and S content are all based on the dry weight of the sludge.

Table S2 Alpha correlation index of archaea and bacteria

| Sample | OTUs | Shannon | Chao1 | Simpson |
| --- | --- | --- | --- | --- |
| Archaea Control-a | 52.6±10.0 | 0.29±0.02 | 8.3±0.5 | 0.64±0.03 |
| Archaea Air-a | 60.3±2.4 | 0.29±0.03 | 9.0±3.6 | 0.66±0.35 |
| Archaea CO2-a | 58.6±4.5 | 0.25±0.01 | 8.3±2.5 | 0.74±0.01 |
| Archaea He-a | 52.0±10.8 | 0.31±0.03 | 8.3±0.7 | 0.63±0.04 |
| Archaea N2-a | 53.6±11.9 | 0.29±0.07 | 7.8±1.4 | 0.68±0.09 |
| Archaea Control-b | 46.6±8.1 | 0.19±0.03 | 10.6±1.1 | 0.81±0.04 |
| Archaea Air-b | 38.3±2.0 | 0.29±0.11 | 9.1±1.2 | 0.68±0.13 |
| Archaea CO2-b | 45.3±14.5 | 0.36±0.07 | 11.1±4.2 | 0.60±0.11 |
| Archaea He-b | 42.3±6.1 | 0.39±0.05 | 11.6±3.3 | 0.53±0.22 |
| Archaea N2-b | 42.7±6.6 | 0.27±0.05 | 9.0±1.0 | 0.72±0.05 |
| Bacteria Control-a | 2724.0±261.2 | 2.51±0.01 | 2724.9±261.1 | 0.0085±0.0015 |
| Bacteria Air-a | 2738.3±55.1 | 2.53±0.04 | 2739.3±55.3 | 0.0086±0.0008 |
| Bacteria CO2-a | 2853.6±227.2 | 2.61±0.03 | 2854.8±227.2 | 0.0074±0.0010 |
| Bacteria He-a | 2861.3±101.4 | 2.41±0.04 | 2651.0±101.4 | 0.0166±0.0008 |
| Bacteria N2-a | 2700.6±122.7 | 2.54±0.04 | 2701.6±122.5 | 0.0083±0.0009 |
| Bacteria Control-b | 2195.0±59.6 | 2.18±0.07 | 2196.3±59.6 | 0.0343±0.0021 |
| Bacteria Air-b | 2232.3±133.5 | 2.24±0.05 | 2233.5±133.7 | 0.0279±0.0012 |
| Bacteria CO2-b | 2351.0±122.5 | 2.18±0.07 | 2352.1±112.5 | 0.0359±0.0132 |
| Bacteria He-b | 2147.1±145.7 | 2.08±0.09 | 2147.9±145.7 | 0.0367±0.0060 |
| Bacteria N2-b | 2347.2±40.0 | 2.08±0.03 | 2348.1±40.3 | 0.0362±0.0005 |

Note: The first ten groups are alpha correlation indices for archaea and the last ten groups are alpha correlation indices for bacteria; ‘-a’ shows microorganisms on Day 8 of anaerobic digestion; ‘-b’ shows microorganisms on Day 35 of anaerobic digestion.