Supplementary Materials for:

Microaerobic condition as pretreatment for improving anaerobic digestion: A review

César Huiliñira*, Jhosané Pagés-Diaza, Gustavo Vargasa, Sylvana Vegaa, Yeney Lauzuriquea and Nicolás Palominosa

Table S.1: Effect of micro- and aerobic pretreatment applied at different substrates for anaerobic digestion.

| Substrates | Pretreatment conditions | Effect/Changes | Result after pretreatment in AD | References |
| --- | --- | --- | --- | --- |
| Organic sludge | Airflow 0.08-0.4 vvm, Continuous during 24 h | High VSS solubilization and volatile matter removal | Production of biogas increased by 1.5 | Hasegawa et al., (2000) |
| Primary sludge | 500 mL/d intermittently 15 seg every min into de sludge phase | Higher hydrolysis of proteins and carbohydrates but no significant influence on the hydrolysis of lipids | Not available | Johansen & Bakke, (2006) |
| Vegetable and flower wastes | Insufficient microaeration (5 min every 24 h) Sufficient microaeration (5 min every 12, 4 and 1 h) | Insufficient microaeration decreased lignocelluloses' biodegradability | -Unstable and decreased performance in AD (with insufficient aeration) -Promotion of the hydrolysis of easily biodegradable carbohydrates and proteins (Sufficient aeration) | Zhu et al., (2009) |
| Primary sludge | Continuously airflow of 0.00156 vvm | Improvement of protein solubilization and 40% tCOD reduction | Not available | Diak et al., (2013) |
| Food waste, yellow and brown water | Continuous, 4 days, 37.5 mLO2/L liquid d | Increased solubilization and acidification efficiencies | Methane yield increased by 10 to 21% | Lim & Wang, (2013) |
| Brown water and food waste | 3 mLair/min/day, 1-2 h after feeding the reactor | More diverse bacterial community | Enhancement of COD solubilization and VFA production | Lim et al., (2014) |
| Synthetic food waste | Semi-continuous time based, 17 days, intermittent 129-387 Lair/kg TS d | Improvement in carbohydrate and protein hydrolysis (21–27% and 38–64% respectively) | Methane yield increased by 5% | Xu et al., (2014) |
| Corn straw | Pulse injection, thermophilic microaerobic (55°C), 5-40 mL O2/g VS substrate | Cellulosic structure was partly disrupted. Crystallinity index decrease | Improvement of the hydrolysis rate. Methane yield increased by 16% | Fu et al., (2015) |
| Olive mill wastewater (OMWW) | 0.65 Lair/L reactor/min (2 L reactor) 5-7 days | 78-90% polyphenols reduction. 18-21% reduction in initial tCOD | -No inhibition in the AD-long-term process using 5-day pretreated OMWW -Higher methane yield (0.39 m3/kg tCOD removed) and tCOD (65%) were achieved for 5-days aeration | González-González & Cuadros, (2015) |
| Corn straw | Pulse injection, 0-100 mL/L liquid d (vvm) | VS removal efficiency up to 54% (10% higher than non-pretreated sample) | Methane yield increased by 17% | Fu et al., (2016) |
| Sewage sludge | Airflow rates=0.1-1vvm, temperatures= 20-35 °C, aeration times= 24-60 h | Improvement of hydrolytic phase. Increase of protein, sugar and sCOD. Better removal of VSS | 211% increase in methane generation (0.3 vvm, 48 h, and 35 °C) | Montalvo et al., (2016a) |
| Mixed sludge | Continuous airflow, 0.1-0.5 vvm, 24-60 h | Increases in soluble protein (185%), total sugars (192%), sCOD (150%) | Methane yield increased by 114% | Montalvo et al., (2016b) |
| Synthetic food waste | Continuously 5 Lair/h, 24 h | Protein and carbohydrate-rich substrates yield the best results regards to lipids-rich material | No positive impact on hydrogen production; however, methane production increased in two-stage AD (35 °C) | Rafieenia et al., (2017) |
| Rice straw | Constant airflow of 0.5 L/min/kg substrates for 8 days at 25, 35 and 45 °C. | Hydrolysis rate was improved | Pre-aerated straw for 2 days /35 °C showed the highest hydrolytic efficiency and methane yield | Zhou et al., (2017) |
| MSW | Pre-aeration of 4-14 days. Initial airflow rate of 10 min followed by a 20 min pause. 0.5 Lair/min/kg waste. | Accelerated degradation of proteins by pre-aeration phase | 8-day pre-aeration obtained the highest biogas yield (192.4 NL/kg DM) with an average methane content of 56% (v/v) | Ni et al., (2017) |
| Napier grass | Pulse injection, 10-15 mL O2/g VS | Increase in the population of facultative microorganisms (attributed) | Enhancement of VFAs yields | Sawatdeenarunat et al., (2017) |
| Wheat straw | Pulse injection, 5-15 mL O2/g VS | Extended hydrolysis and solubilization of straw | Methane yield increased by 7% with the most efficient microaeration strategy (5 mL O2/g VS for 3 days) | Tsapekos et al., (2017) |
| Synthetic oleate wastewater | Open to air | Stimulation of the development of facultative bacteria | Lower oleate and higher palmitate levels were advantageous for methane production | Duarte et al., (2018) |
| Food waste | 5-21% O2; (0.0167 vmm), 1-4 days | Increment in hydrolytic exoenzymes and water-soluble macromolecules | Microbial community change from lactic acid bacteria to VFAs consumers and Fungi (at 21%O2) | Fisgativa et al., (2018) |
| Waste activated sludge | Continuous 24 h, 0.03-0.11 vvm, 70 °C | Improvement in sludge solubilization, organic matter dissolution and growth of microbial species (i.e., Lactococcus) | Not available | Hou et al., (2018) |
| # Kitchen waste | Continuous aeration 0.19–0.22 vvm, 44 - 48 °C, pH 7.2–7.5 | Reduction in ammonia accumulation | Higher methane yield (%) in thermophilic digester compared to the mesophilic digester. The higher stability of thermophilic digester due to lower population of Treponema sp | Sahu et al., (2019) |
| Mixed sludge | Continuous 0-6 Lair/g TS min, 0-48 h | Release of dissolved organic matters, generation of VFAs and stimulation of enzyme activity | -Promoted enrichment of hydrolytic and fermentative bacteria -Biogas production increased by 18% with methane content enhancement | Ruan et al., (2019) |
| Olive mill wastewater | Temperature treatment= 25–28 °C, air flow rate of 0.5 L/L substrate/min, 2-6 days | High VS, tCOD and polyphenols removal efficiency | Highest biogas production at 4 days aeration (i.e., 0.437 Nm3/kg VS). | Alrawashdeh & Al-Essa, (2020) |
| MSW | 254 Lair/kg TS d, 15 min every 2 h | Reduction in hydrolysis time and increment in organic matter solubilization | Increase in enzymatic activity | Bacab et al., (2020) |
| Syntethic coal gasification wastewater | Open to air, continuous 12 h | Not available | Lower removal of COD and total phenols in AD | Chen et al., (2020b) |
| Food waste | -2 days’ dynamic aeration (50 Lair/h), 39 °C -2 days’ static storage, room temperature | Not available | -Stable production of VFAs along the digestion process and enhanced methane production (at 2 days’ aeration step) -Rapid acidification and significant decrease of methane production rate (at 2 days’ storage step) | Fisgativa et al., (2020) |
| Raw sludge (30% primary + 70% secondary) | Continuous 0.1-0.5 vvm, 20-60 h | Hydrolysis rate improvement (i.e., sCOD increased by 65%) | Improvement in methane production (221%) | Rashvanlou et al., (2020) |
| Poultry manure | 0.2 vvm, 10 times daily during 15 min | Decrease in VS and total organic carbon content | -CH4 concentration in biogas increased from 55% to 63% -NH3 emission decreased up to 34% | Rubezius et al., (2020) |
| Oilseed rape straw | Continuous 20-120 mL O2/g VS d for 10 days | 120 mL O2/g VS d increased the degradation of cellulose, hemicellulose, and lignin content | Not available | Wang et al., (2020) |
| Blackwater (toilet wastewater) | Pulse injection, 5-150 mg O2/L reactor cycle | Increase in hydrolysis rate (from 35 to 49%) | -Higher abundance of hydrolytic/fermentative bacteria and acetoclastic methanogens (at low microaeration doses) -Enhanced methane production (5 mg O2/L reactor d) | Yu et al., (2020) |
| Kitchen waste | Pulse injection, 5-20 mL O2 | Decomposition of nitrogenous molecules (at 20 mL O2) | Peak gas production (mL) increased by 21% (at 20 mL O2) | Zhen et al., (2020) |
| Piggery waste | Controlled ORP to -420 mV | Better hydrolysis efficiency (~50%) | Increase in methane yield as high as 20% | Chetawan et al., (2021) |
| Corn stover | Open to air | Promoted hydrolysis and acidogenesis phases | 12 h pretreatment at pH=8 before AD increased the methane yield by 17% | Xu et al., (2021) |
| Raw sludge (30% primary + 70% secondary) | Continuous 0.1-0.5 vvm, 20-60 h, 38 °C | -Highest solubilization of sCOD (65%) at 0.2vvm, 40 h -Slight increase in enzyme activity (3%) -No significant change in lipase activity | Methane production increased by 148% | Rashvanlou et al., (2021) |
| Corrugated board and tissue paper | Pulse injection, 5-30 mLO2/g VS added | Biodegradability increased by 79% | The highest methane yield achieved at 15 mL O2/g VS | Song et al., (2021) |
| Blackwater (toilet flush) | 5-150 mg O2/g COD per cycle | Syntrophic bacteria and methanogens were enriched at low O2 dosage | Oxygen dosage influenced the prevalence of the population carrying antimicrobial resistance genes | Guo et al., (2021) |
| Food waste and sewage sludge | Aerobically incubated for 6 days, 30 °C, manually stirred every 24 h | Ammonium released after pre-aeration | -CH4 productivity increased by 60% -Microbial community distribution was changed -Methanosarcina abundance was over 93% | Cheng et al., (2021) |
| Rice straw, pig manure | Continuous 14 days, 1 Lair/kg min. Combined aerobic + microaerobic | Improvement in the ability of microorganisms to degrade substrate and hydrolyzed big particle size | Not available | Xiang et al., (2022) |
| Pinewood | Open to air, 10 days | Not available | Methane yield improved by 7.3 folds | Karami et al., (2022) |
| Corn straw | Pulse injection, 0.05-1.6 mL/g VS d | Improvement in VFAs concentration, cellulase activity, and soluble humic acid concentration | -AD system showed higher electron-transfer capacity -Shortened the lag phase and methane yield increase by 8% (at 0.2 mL/g VS d) -Bacterial diversities and relative abundances of facultative and acetate-oxidizing bacteria increased | Zhu et al., (2022a) |
| Swine manure and corn silage | 1vvm, intermittent microaeration for 5 min, 35 °C | Enhanced VFAs (20 %) and lactic acid (19%) production | Increased maximum VFAs concentration | Cao et al., (2022) |
| Waste activated sludge | Continuos 48 h, 0.35 vvm | Increase in sCOD (32%) and soluble proteins (39%) | -Better performance and stability of AD digester -Improvement in methane production, VSS degradation and sCOD degradation | Castillo et al., (2022) |
| Poultry litter (PL) and wheat straw (WS) | Pulse Injection 1-4 days, 120 NmL air/d | Not available | Daily methane production increases by 15% | Zhan et al., (2023) |

Table S.2: Comparison of the relative abundance of microorganisms found in reactors with microaeration and without microaeration treating different substrates.

| Substrate | Phylum | Gender | Class | Relative abundance (%) no microaeration | Relative abundance (%) with microaeration | References |
| --- | --- | --- | --- | --- | --- | --- |
| Brown water and food waste | Firmicutes Bacteroidetes |  |  | 58 0 | 72 24 | Lim et al., (2014) |
| Corn straw | Firmicutes |  | Clostridia | 81 81 | 86 85 | Fu et al., (2016) |
| Swine manure |  | Clostridium Terrisporobacter |  | Day 1-10: 37 - 58 Day 1-10: 7 - 12 | Day 1-10: 30 – 61 Day 1-10: 7 - 11 | Cao et al., (2022) |
| Food waste | Proteobacteria |  |  | Day 27: <3 | Day 27: 49 | Fisgativa et al., (2018) |
| Blackwater | Bacteroidetes Proteobacteria Firmicutes |  |  | 45 16 14 | 45 16 15 | Guo et al., (2021) |
| Food waste | Firmicutes Actinobacteriota | Methanosaeta |  | <70 ≈15 ≈15 | >93 ≈30 ≈27 | Li et al., (2022) |
| Mixed sludge | Proteobacteria Chloroflexi Bacteroidetes Parcubacteria Synergistetes Actinobacteria Firmicutes | Methanosaeta Methanobacterium Methanobrevibacter |  | 22 13 10 2 4 4 4 66 8 3 | 25 16 11 11 7 7 6 68 9 4 | Ruan et al., (2019) |
| Corrugated board and tissue paper (microbial agent: sheep manure) |  | Bacteroides Macellibacteroides Acinetobacter |  | 0.63 0.15 16 | + dozens of times + hundreds of times 31 | Song et al., (2021) |
| Blackwater | Bacteroidetes | Methanosarcina Methanosaeta |  | 37 2 <40 | 45 11 >40 | Yu et al., (2020) |
| Food waste and sewage sludge |  | Methanosarcina |  | 62 | 96 | Cheng et al., (2021) |
| Corn straw | Bacteroidota Firmicutes Synergistota Spirochaetota |  |  | <60 - - 1 | <30 + 50.19 + 91.42 6 | Zhu et al., (2022a) |

(+) represents the percentage of times that the relative abundance of a microorganism is higher with microaeration than without microaeration.

Table S.3: Variation of profit (€/kg TS added) at different solid concentrations. Time of aeration = 48 h; Percentage of CH4 increasing = 50%.

| Condition | Solid concentration (g TS/L) | QPT (kWh/kg TSadded) | EMA (kWh/kg TSadded) | QAD (kWh/kg TSadded) | Energy Output, (kWh/kg TSadded) | Net Energy (kWh/kg TSadded) | Profit (€/kg TSadded) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| With pretreatment | 20 | 0.754 | 2.835 | 0.116 | 6.04 | 2.335 | 0.467 |
| without pretreatment | 20 | --- | --- | 0.87 | 4.02 | 3.157 | 0.631 |
| With pretreatment | 30 | 0.503 | 1.89 | 0.077 | 6.04 | 3.57 | 0.714 |
| without pretreatment | 30 | --- | --- | 0.58 | 4.02 | 3.45 | 0.689 |
| With pretreatment | 50 | 0.302 | 1.134 | 0.0464 | 6.04 | 4.56 | 0.912 |
| without pretreatment | 50 | --- | --- | 0.3484 | 4.02 | 3.68 | 0.735 |
| With pretreatment | 75 | 0.201 | 0.756 | 0.031 | 6.04 | 5.05 | 1.011 |
| without pretreatment | 75 | --- | --- | 0.232 | 4.02 | 3.79 | 0.759 |

Table S.4: Variation of profit (€/kg TS added) at different aeration times. Solid concentration = 50 g/L; Percentage of CH4 increasing = 50%.

| Condition | Aeration time (d) |  | QPT (kWh/kg TSadded) |  | EMA (kWh/kg TSadded) |  | QAD (kWh/kg TSadded) |  | Energy Output, (kWh/kg TSadded) |  | Net Energy (kWh/kg TSadded) |  | Profit (€/kg TSadded) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| With pretreatment | 2 | 0.302 |  | 1.134 |  | 0.0464 |  | 6.04 |  | 4.559 |  | 0.912 |  |
| without pretreatment | 2 |  |  |  |  | 0.3484 |  | 4.02 |  | 3.679 |  | 0.736 |  |
| With pretreatment | 2.5 | 0.302 |  | 1.4175 |  | 0.0464 |  | 6.04 |  | 4.275 |  | 0.755 |  |
| without pretreatment | 2.5 |  |  |  |  | 0.3484 |  | 4.02 |  | 3.679 |  | 0.736 |  |
| With pretreatment | 3 | 0.302 |  | 1.701 |  | 0.0464 |  | 6.04 |  | 3.992 |  | 0.798 |  |
| without pretreatment | 3 |  |  |  |  | 0.3484 |  | 4.02 |  | 3.679 |  | 0.736 |  |
| With pretreatment | 4 | 0.302 |  | 2.268 |  | 0.0464 |  | 6.04 |  | 3.425 |  | 0.685 |  |
| without pretreatment | 4 |  |  |  |  | 0.3484 |  | 4.02 |  | 3.679 |  | 0.736 |  |

Table S.5: Variation of profit (€/kg TS added) at different increasing CH4 percentages. Solid concentration = 50 g/L; aeration time = 2 d.

| Condition | Methane increasing (%) | QPT (kWh/kg TSadded) | EMA (kWh/kg TSadded) | QAD (kWh/kg TSadded) | Energy Output, (kWh/kg TSadded) | Net Energy (kWh/kg TSadded) | Profit (€/kg TSadded) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| With pretreatment | 20 | 0.302 | 1.134 | 0.0464 | 4.833 | 3.351 | 0.67 |
| without pretreatment | 20 |  |  | 0.3484 | 4.02 | 3.679 | 0.736 |
| With pretreatment | 30 | 0.302 | 1.134 | 0.0464 | 5.236 | 3.753 | 0.751 |
| without pretreatment | 30 |  |  | 0.3484 | 4.02 | 3.679 | 0.736 |
| With pretreatment | 50 | 0.302 | 1.134 | 0.0464 | 6.04 | 4.559 | 0.912 |
| without pretreatment | 50 |  |  | 0.3484 | 4.02 | 3.679 | 0.736 |
| With pretreatment | 75 | 0.302 | 1.134 | 0.0464 | 7.048 | 5.566 | 1.113 |
| without pretreatment | 75 |  |  | 0.3484 | 4.02 | 3.679 | 0.736 |