Table A.1. Physicochemical characteristics of dairy wastewater.

| Parameters |  |
| --- | --- |
| pH | 6.63 ± 0.1 |
| TDS (mg/L) | 3115 ± 8 |
| EC (ms/cm) | 4.935 |
| TSS (mg/L) | 55 |
| COD (mg/L) | 3850 ± 10 |
| BOD (mg/L) | 1900 ± 5 |
| Oil & Grease (mg/L) | 78.8 ± 0.1 |

Table A.2. Optical density of maltose by spectrophotometry

| Optical density (OD) | Concentration (μmol/mL) | Distilled water (mL) | Stoke maltose solution (mL) |
| --- | --- | --- | --- |
| 0 | - | 5 | Control |
| 0.094 | 0.5 | 4.82 | 0.18 |
| 0.178 | 1 | 4.7 | 0.3 |
| 0.614 | 2.5 | 4.1 | 0.9 |
| 2.323 | 5 | 3.2 | 1.8 |
| 2.92 | 7.5 | 2.3 | 2.7 |
| - | 10 | 1.4 | 3.6 |

Fig. A.1. The maltose standard curve.

Table A.3. Optical density of L-tyrosine by spectrophotometry

| Optical density (OD) | Concentration (μmol/mL) | Distilled water (mL) | Stock L-tyrosine solution (mL) |
| --- | --- | --- | --- |
| 0 | - | 2 | Control |
| 0.041 | 0.055 | 1.95 | 0.05 |
| 0.189 | 0.111 | 1.9 | 0.1 |
| 0.353 | 0.221 | 1.8 | 0.2 |
| 0.683 | 0.442 | 1.6 | 0.4 |
| 0.819 | 0.553 | 1.5 | 0.5 |

Fig. A.2. The L-tyrosine standard curve.

Table A.4. Cumulative biogas production in anaerobic biological reactors loaded with sludge and pretreated with to different ratios of enzyme solution extracted from the organic waste (R1, R2, R3 and R4, control).

|  | % Enzyme solution | Cumulative gas production |
| --- | --- | --- |
| Control | 0 | 6565 |
| R1 | 5 | 7095 |
| R2 | 10 | 8100 |
| R3 | 15 | 6900 |