Supporting Information

Upflow anaerobic-microaerobic fixed biofilm reactor integrating methanogenesis with partial nitrification

Abid Hussain, Jangho Lee, Robertson Reid, Hyung-Sool Lee*

Department of Civil and Environmental Engineering, University of Waterloo, 200 University Avenue West, Ontario, Canada, N2L3G1

*Corresponding author: Phone: +1-519-888-4567 Ext. 31095; Fax: +1-519-888-4349;   E-mail: hyungsool@uwaterloo.ca

Estimation of Oxygen Supply Rate (OSR) through the gas membrane

OSR through the membrane was evaluated at different air pressures of 14, 28 and 41 kPa. A cylindrical vessel similar in design and volume to the microaerobic zone of the AMFB reactor was filled with distilled water that had been deoxygenated by continuous sparging of N2 gas (99%). A DO meter was used for continuous monitoring of dissolved oxygen in the test vessel. Membrane aeration was initiated once a DO of < 0.1 mg/L was reached. The change in DO concentration with active membrane aeration was monitored with the DO meter and used for estimation of OSR according to equations (1) and (3) below [1, 2].

Oxygen transfer through the membrane can be described by the first order differential equation [1]:

dCdt= KLa (Cct- Ct)							(1)

Where: Ct is the oxygen concentration in the liquid phase at any given time t (mg/L), Cct is the saturated oxygen concentration (mg/L),  KL is the mass transfer coefficient based on liquid phase (m/h), t is the aeration time (h), a is the interfacial area available for mass transfer based on membrane surface area A (m2) and liquid volume V (m3).

Assuming that Cct is constant at all times, there is no chemical reaction, and initially (at t = 0) there is no oxygen in the test vessel, the solution of equation 1 yields:

ln Cct-C0Cct-Ct = KLa (t)								(2)

Where: C0 is the initial dissolved oxygen concentration in the liquid phase (mg/L)

Equation 2 corresponds to a linear relationship, in which the slope of the line is the volumetric mass transfer coefficient, KLa [1-3]. Figure 1 shows the KLa obtained for the membrane at different pressures in this study. The interfacial area, a can be deduced from the membrane surface area and liquid volume of the test vessel (A/V). OSR can then be calculated by the following equation [1]:

OSR = KL X Cct X 1000								(3)

Where: OSR is the oxygen supply rate (mg/m2 h), 1000 is the unit conversion coefficient.

(B)

(A)

(C)

Figure 1. C* versus time for membrane at pressures of (A) 14 Kpa (B) 28 Kpa (C) 41 Kpa. Where: C* = ln Cct-C0Cct-Ct, Ct is the oxygen concentration in the liquid phase at time t (mg/L), Cct is the saturated oxygen concentration (mg/L),  C0 is the initial dissolved oxygen concentration in the liquid phase (mg/L). The slope of the line is the volumetric mass transfer coefficient KLa.

References

[1] W-Y. Dong, HJ. Wang, WG. Li, WC. Ying, GH. Gan, Y. Yang, Effect of DO on simultaneous removal of carbon and nitrogen by a membrane aeration/filtration combined bioreactor, J. Membrane Sci. 344 (2009) 219-224.

[2] P.C. Munasinghe, S.K. Khanal, Syngas fermentation to biofuel: Evaluation of carbon monoxide mass transfer coefficient (kLa) in different reactor configurations, Biotech. Prog. 26 (2010) 1616-1621.

[3] A. Hussain, B. Tartakovsky, S.R. Guiot, V. Raghavan, Use of silicone membranes to enhance gas transfer during microbial fuel cell operation on carbon monoxide, Bioresour. Technol. 102 (2011) 10898-10906.