Supporting Information

Intermittent air mixing enhances dry anaerobic co-digestion of pig manure solid fraction and wheat straw: methane performance and machine-learning-guided parameter optimization

Table：S1-4

Figure：S1

Table S1. Physicochemical characteristics of the substrates and inoculum

| Item | Wheat straw | Pig manure solid fraction | Inoculum sludge |
| --- | --- | --- | --- |
| pH | / | 7.4±0.2 | 7.1±0.1 |
| Total solid (TS, %) | 90.6±0.5 | 24.0±0.4 | 20.8±0.1 |
| Volatile solid (VS, %) | 77.4±0.9 | 19.8±0.2 | 8.9±0.3 |
| VS/TS | 0.85 | 0.83 | 0.43 |
| Total chemical oxygen demand (TCOD, g L-1) | / | 312±19 | / |
| Soluble chemical oxygen demand (SCOD, g L-1) | / | 39±11 | / |
| C/N | 29.5 | 14.0 | / |

Table S2. Experimental design for anaerobic digestion with different PMSF-to-straw ratios

| Item | Blank group | Treatment Groups |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| PMSF: straw ratio |  | 100:0 | 75:25 | 50:50 | 25:75 | 0:100 |
| C/N of mixed feedstock | / | 14.0 | 17.9 | 21.8 | 25.6 | 29.5 |
| PMSF TS added (g TS) | 0 | 34.5 | 25.9 | 17.3 | 8.6 | 0 |
| PMSF VS added (g VS) | 0 | 29.6 | 21.1 | 13.3 | 6.3 | 0 |
| Straw TS added (g TS) | 0 | 0 | 8.6 | 17.3 | 25.9 | 34.5 |
| Straw VS added (g VS) | 0 | 0 | 7.0 | 13.3 | 18.8 | 23.6 |
| PMSF fresh mass (g) | 0 | 150.0 | 112.5 | 75.0 | 37.5 | 0.0 |
| Straw fresh mass (g) | 0 | 0 | 7.8 | 15.7 | 23.5 | 31.4 |
| Inoculum added (g) | 150.0 | 150.0 | 150.0 | 150.0 | 150.0 | 150.0 |
| Inoculum TS added (g TS) | 25.5 | 25.5 | 25.5 | 25.5 | 25.5 | 25.5 |
| Deionized water added (g) | 150.0 | 0 | 29.7 | 59.3 | 89.0 | 118.6 |
| Total working mass (g) | 300.0 | 300.0 | 300.0 | 300.0 | 300.0 | 300.0 |

Table S3. Operating conditions for the semi-continuous parameter optimization experiment

| No. | TS (%) | HRT (d) | OLR (g TS L-1 d-1) | OLR (g VS L-1 d-1) | Daily out/in (g d-1) | Straw (g d-1) | PMSF (g d-1) | Water (g d-1) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 20 | 25 | 8.0 | 6.7 | 12.1 | 1.3 | 5.0 | 5.8 |
| 2 | 20 | 35 | 5.7 | 4.8 | 8.5 | 0.9 | 3.6 | 4.0 |
| 3 | 20 | 45 | 4.4 | 3.7 | 6.5 | 0.7 | 2.8 | 3.0 |
| 4 | 25 | 25 | 10.0 | 8.4 | 11.9 | 1.7 | 6.2 | 4.0 |
| 5 | 25 | 35 | 7.1 | 6.0 | 8.7 | 1.2 | 4.5 | 3.0 |
| 6 | 25 | 45 | 5.6 | 4.7 | 6.7 | 0.9 | 3.5 | 2.3 |
| 7 | 30 | 25 | 12.0 | 10.1 | 12.5 | 2.0 | 7.5 | 3.0 |
| 8 | 30 | 35 | 8.6 | 7.2 | 8.8 | 1.4 | 5.4 | 2.0 |
| 9 | 30 | 45 | 6.7 | 5.6 | 6.8 | 1.1 | 4.2 | 1.5 |

Table S4. Representative sample groups for microbial community sequencing

| Sample code | Air mixing intensity (mL L-1 min-1) | Loading level | TS (%) | HRT (d) | OLR (g TS L-1 d-1) |  | OLR (g VS L-1 d-1) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| A0-L | 0 | Low | 20 | 45 | 4.4 |  | 3.7 |
| A0-M | 0 | Medium | 25 | 35 | 7.1 |  | 6.0 |
| A0-H | 0 | High | 30 | 25 | 12.0 |  | 10.1 |
| A33.3-L | 33.3 | Low | 20 | 45 | 4.4 |  | 3.7 |
| A33.3-M | 33.3 | Medium | 25 | 35 | 7.1 |  | 6.0 |
| A33.3-H | 33.3 | High | 30 | 25 | 12.0 |  | 10.1 |
| A66.7-L | 66.7 | Low | 20 | 45 | 4.4 |  | 3.7 |
| A66.7-M | 66.7 | Medium | 25 | 35 | 7.1 |  | 6.0 |
| A66.7-H | 66.7 | High | 30 | 25 | 12.0 |  | 10.1 |

Note: A0, A33.3, and A66.7 represent air mixing intensity of 0-, 33.3-, and 66.7-mL L-1 d-1, respectively; L, M, and H represent low-, medium-, and high-OLR sequencing samples.

Figure S1. Volumetric methane production rate (a-b) and methane content (b) of anaerobic co-digestion of PMSF and straw under different OLR and intermittent air mixing intensities