Supplementary materials

of

Upgrade the high-load anaerobic digestion and relieve acid stress through the strategy of side-stream micro-aeration: biochemical performances, microbial response and intrinsic mechanisms

Wen Lia, Yongli Liua, Baocun Wua, Li Gua*, Rui Dengb

- Key laboratory of the Three Gorges Reservoir Region’s Eco-environments, Ministry of Education, Institute of Environment and Ecology, Chongqing University, 174 Shapingba Road, Chongqing, 400045, P.R. China

- School of Architecture and Urban Planning, Chongqing Jiaotong University, Chongqing 400074, P.R. China

Table S1 Characteristics of FW and inoculated sludge

| Characteristics | Feedstock | Inoculum |
| --- | --- | --- |
| TS (g/L) | 125.70± 2.80 | 76.00±1.30 |
| VS (g/L) | 120.90±3.00 | 27.40±0.50 |
| VS/TS(%) | 97.60 | 25.60 |
| pH | 4.90 | 7.50 |
| TCOD (mg/L) | 162833±4249 | 37941±2517 |
| SCOD (mg/L) | 48240±1400 | 1651±327 |
| Crude fat(g/L) | 20.03g±0.50 | NA |
| Crude protein(g/L) | 18.40±1.70 | NA |
| Carbohydrate(g/L) | 70.36±4.00 | NA |
| VFA/ALK (g acetate equivalent/g CaCO3 equivalent) | NA | 0.10 ± 0.00 |

Table S2a Experimental design and analysis performed

| Groups | Experimental design |  |  |  | Analysis |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Structing |  | Micro-aeration |  | Enzymes |  | C-blance |  | CLSM |  | Microbial and metagenomic analysis |
|  | - |  | mL/d |  |  |  |  |  |  |  |  |
| DMA | 0.7 L digester |  | 20 |  | N/A |  | N/A |  | N/A |  | N/A |
| SD | 3 L main digester coupled with a 0.7 L side-stream reactor |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |
| SMA |  |  | 20 |  | OLR 5.0 gVS/L·d, before and after the introduction of micro-aeration |  |  |  |  |  |  |

Table S2b Operational conditions for different stages in micro-aeration participated groups

|  | SRT (d) | OLR (g VS/L) | Time (day) | Aeration intensity (mL/d) |
| --- | --- | --- | --- | --- |
| Stage I | 20 | 2.5 | 10-25 | N/A |
| Stage II |  | 4.0 | 26-45 |  |
| Stage III |  | 5.0 | 46-65 |  |
| Stage IV |  | 5.0 | 66-90 | 20 |
| Stage V |  | 5.5 | 91-110 |  |

Table S2a describes the experimental design and analysis of the three groups of reactors. First, three groups of reactors were used as controls for structures and micro-aeration to explore the role of micro-aeration in the side-stream reactor. In addition, enzymes, C-blance, CLSM, microbial and microbial and metagenomic analysis were performed before and after the introduction of micro-aeration to further explore the intrinsic mechanisms of SMA.

Table S2b describes the operational conditions for different stages in micro-aeration participated groups, with OLR gradually increasing from 2.5 to 5.5 g VS/L, while changing the micro-aeration situation at the OLR of 5.0 g VS/L. The analysis of the effect of micro-aeration on the system focused on the OLR of 5.0 g VS/L.

Figure S1 Schematic diagram of the device structure

Micro-aeration dose

Sludge run in a single-stage reactor to acidification was transferred to four reactors under an organic load of 5.0 g VS/L. R1, R2, R3, R4 and side-stream reactors have the same structure with the effective volume of 0.7 L. Different doses of micro-aeration (5, 10, 20 and 30 mL/d) were applied in these four reactors respectively and their operational performance is shown in Figure S2. The results in direct micro-aeration with different gas dosage give us the right dosage in SMA research, and 20 mL/d was selected as the optimal dose.

Figure S2 Digestive performance under the effect of DMA with different air dosage under OLR of 5.0 g VS/L

Figure S3 Changes of VFA/Alk ratio in SMA. The transparent part is the error band

Figure S4 Changes of VFA component in SMA (mg HAc/L)

Figure S5 ORP variation in SMA and DMA digester during one day digestion

Figure S6 Relative abundance of bacteria at the phylum level (a) and archaea at the genus level (b) in SMA reactors and inoculum sludge samples

Figure S7 Changes in the abundance of genes associated with methanogenesis (Read number)

Corresponding author: Key Laboratory of the Three Gorges Reservoir Region’s Eco-Environment, Ministry of Education, Chongqing University, Chongqing, PR China.

School of Architecture and Urban Planning, Chongqing Jiaotong University, 66 Xuefu Avenue, Chongqing, 400074, P.R. China

Email address: guli@cqu.edu.cn (Li Gu); Trdeng@sina.com (Rui Deng)