Supplementary Information

Reactive Oxygen Species Generation Induced by Coupling Phanerochaete chrysosporium with Intermittent Micro-aeration to Enhance Methane Production

Xuepeng Wanga, Zhenru Wanga, Ying Denga, Haohao Maoa, Qilin Yua, Yaobin Zhanga*

Affiliations:

a Dalian University of Technology School of Environmental Science and Technology No.2 Linggong Road, Ganjingzi District. Dalian, Liaoning, 116024, China.

* Correspondence: Tel: +86 411 8470 6460, Fax: +86 411 8470 6263;

E-mail address: zhangyb@dlut.edu.cn.

Text S1 The selection of the intensity of intermittent aeration

In the previous study, a high methanogenic performance was observed at an aeration intensity of 7%(VAeration:VReactor)/day, and the abundance of methanogens and antioxidant enzymes-encoding genes in response to oxidative stress significantly increased, enabling methanogenesis to proceed smoothly under intermittent microaeration [1]. In a pure culture experiment, it is observed that the spore suspension of white-rot fungi could grow under aeration intensities of 4%(VAeration:VReactor)/day), 8%(VAeration:VReactor)/day), and 12%(VAeration:VReactor)/day). Increasing the aeration intensities was beneficial to the growth of white-rot fungi (unpublished data). In the present study, in Stage 1 and 2, a conservative aeration intensity (20 mL/day (4%(VAeration:VReactor)/day)) was adopted to enable methanogens to adapt to the micro-aerobic environment. In Stage 3, the aeration intensity was further increased to 40 mL/day (8%(VAeration:VReactor)/day) to promote fungal proliferation. The digestion performance and metagenomic results showed that this aeration intensity did not damage the functioning of the anaerobic microbial community. Although the aeration intensity of 12%(VAeration:VReactor)/day was more favorable for fungal proliferation, the introduction of a large amount of air would reduce the methane proportion in biogas (by more than 5%), which was unfavorable for subsequent purification. Therefore, the aeration intensity was not further increased.

Text S2 ROS measurement

The parameters of electron paramagnetic resonance (EPR) spectrometer

EPR instrument (Bruker, Elexsys II E500-CW-EPR) operating parameters were center field 3500 G; microwave frequency 9.8 GHz; microwave power 2.0 mW; modulation frequency 4.0 G; modulation amplitude 4.0 G; sweep width 200 G; receiver gain 3.54 × 104; time constant 41.0 ms; sweep time 15 s; 15 scans.

Quantitative analysis of •OH

The generation of •OH was measured using a fluorescence probe method with 100 µM coumarin [2]. After the reaction, a 0.5 mL sample was collected, and 1 mL methanol was added to terminate the reaction. The sample was centrifuged and filtered through a 0.22 µm membrane. The oxidation product of coumarin by •OH, 7-hydroxycoumarin (7h-COU), was measured using a microplate reader with an excitation wavelength of 350 nm and an emission wavelength of 460 nm. The •OH concentration was then calculated based on 7h-COU/14.5%.

Quantitative analysis of H2O2

The H2O2 concentration was measured using the cerium sulfate titration method where the yellow Ce4+ solution would be reduced by H2O2 to colorless Ce3+ following the stoichiometry of 2Ce4+ + H2O2 = 2Ce4+ + 2H+ + O2. The concentration of Ce4+ before and after the redox reaction can be detected by ultraviolet-visible spectroscopy. The test value at 316 nm was adopted. 1 mM of Ce4+ solution was prepared by dissolving 332 mg of Ce(SO4)2 in 0.5 M of H2SO4 solution(1L). The H2O2 concentration is calculated by the equation of

C(H2O2)= (C0(Ce4+)×V(Ce4+) - C(Ce4+)×(V(Ce4+)+V(H2O2)))/(2×V(H2O2))

Text S3 Analytical methods

The determination of TSS, VSS

Briefly, approximately 5-10 g of thoroughly homogenized sample was transferred into a pre-weighed, pre-combusted porcelain crucible and dried at 105°C for 6 h to determine TSS. After cooling in a desiccator for 30 min, the crucible was weighed, and the same sample was subsequently combusted at 550°C for 2 h in a muffle furnace to determine VSS. TSS content was calculated as the percentage of residue remaining after drying relative to the initial sample weight, while VSS content was determined as the percentage of weight loss during combustion relative to the initial sample weight. All measurements were performed in triplicate using an analytical balance with a precision of ±0.1 mg, and blank controls were included to ensure measurement accuracy. Before analysis, samples were carefully homogenized to ensure representativeness, and any large particles were crushed or ground to achieve a uniform consistency. This method ensures reliable quantification of solid fractions in sludge and cow manure samples.

The determination of biogas content and volume

The content of biogas was measured with a gas chromatograph (Tianmei, GC-7900, China) with a thermal conductivity detector (TCD) [3] before the volume was measured with a 50 mL glass syringe.

Text S4 Electrochemical measurement

After washing three times with 0.01 M phosphate buffer solution (PBS), 20 μL of the sediment suspensions were coated on a glassy carbon electrode (Φ3 mm, working area: 0.07 mm2) and dried in an infrared oven. Electrochemical tests were performed with an electrochemical workstation (Chenhua, CHI 660). The coated glassy carbon electrode, Ag/AgCl electrode, and Pt electrode were selected as the working electrode, reference electrode, and counter electrode, respectively. The capacitance and resistance of the sediment were assessed with cyclic voltammetry (CV) and electrochemical impedance spectroscopy (EIS), respectively.

Cyclic voltammetry was performed in the range of -1.0 to 1.0 V (vs Ag/AgCl) at scan rates of 50 mV/s, with 30 mL of 0.01 M PBS as the electrolyte. Electrochemical impedance spectroscopy was conducted over a frequency range of 1000 kHz to 0.01 Hz, with a sinusoidal perturbation of 10 mV amplitude to analyze the internal impedances of the sediments. Before the electrochemical tests, the electrolyte in the same concentration was aerated for 40 min with N2 to maintain an anaerobic condition.

Text S5 Determination of Cellulose, Hemicellulose, and Lignin

The contents of cellulose, hemicellulose, and lignin in the sludge samples were determined using the van Soest method with a semi-automatic fiber analyzer(F1600, Alva, China)[4]. The samples were air-dried or oven-dried at 45°C and passed through a 1 mm sieve. After numbering the filter bags, they were dried at 105°C for 1 hour. The pretreated filter bags were weighed, and a suitable amount of sample was placed in the filter bags and weighed, with empty filter bags serving as blanks. The sealed filter bags were then subjected to a stepwise washing procedure.

Text S6 Measurement of enzyme activity

Laccase (Lac), a copper-containing polyphenol oxidase, decomposes the substrate ABTS to produce ABTS radicals, exhibiting a significantly greater absorbance coefficient at 420 nm compared to the substrate (ml076971, 100T/48S). The rate of increase in these radicals is used to determine laccase activity. In contrast, lignin peroxidase (LiP), a ferrous heme-containing peroxidase, catalyzes the demethylation of asphaltene, resulting in a decrease in absorbance value measured at 651 nm (ml092790, 100T/96S). Likewise, manganese peroxidase (MnP), another ferrous heme-containing enzyme, performs the oxidation of guaiacol to tetra-o-methoxylenol, displaying a distinct absorption peak at 465 nm (ml076964, 100T/96S) [5].

Text S7 Microbial community analysis

Fungal community

At the end of each stage, 5 mL of sludge samples were collected and frozen at -80℃ before microbial community analysis. DNA was extracted from sludge samples using a rapid DNA extraction kit (Bioteke Corporation, China). For fungal community analysis, the ITS1-ITS2 regions were amplified using primers ITS1F/ITS2R [5]. The purified PCR products were sequenced by the Illumina Hiseq 2000 platform (Shanghai Lingen Biotechnology Co., Ltd.).

Metagenomic analysis

At the end of the operation, 5 mL of the sludge samples were collected and frozen at -80℃ before metagenomic analysis. Nine random DNA sequences were synthesized, and their base arrangement was different from that of microbial gene sequences in nature. In the experiment, 9 sequences were set into 4 gradients according to the copy number content, and the multiple of each gradient was 10-fold. These sequences were designed and synthesized by GeneCowin Biotechnologies Inc., Shanghai, China.

The metagenomic absolute quantification sequencing was performed by GeneCowin Biotechnologies Inc., Shanghai, China. Briefly, total genomic DNA was extracted using the FastDNA® SPIN Kit for Soil (MP Biomedicals, Santa Ana, CA) according to the manufacturer's instructions. The integrity of genomic DNA was detected through agarose gel electrophoresis, and the concentration of genomic DNA were detected through the Qubit3.0 Spectrophotometer. The DNA extracts were spiked with internal standard spike-in DNA（Genesky, China） at 0.1% total DNA by mass. The DNA after mixing internal standard spike-in DNA was sheared by Covaris® system (Covaris, USA). DNA libraries were prepared by a ligation dependent method by using DNA library preparation kit (Genesky, China). 500-600 bp range of sequencing library was size-selected and recovered using MagicPure® Size Selection DNA Beads (Transgen，Beijing). DNA libraries were qualified on the Agilent 2100 Bioanalyzer (Agilent, USA), and quantified with Kapa Library Quantification kit (Kapa Biosystems, Wilmington, MA) using the ABI 9700HT Fast Real-Time PCR system (Thermo Fisher Scientific). The libraries were sequenced on NovaSeq 6000 platform (Illumina USA) and generated 10G bps of 2×150bp pair-end reads.

In the sequencing data, internal standard spike-in DNA sequence was first identified and moved out to make the standard curve. Standard curve of each sample was made with the logarithmic value of read-count per unit length as the horizontal coordinate, and the copy number logarithmic value of each gradient sequence in the internal standard spike-in DNA sequence as the vertical coordinate. Based on the standard curve, the absolute copy number of each gene fragment in the sample was quantitatively calculated

Fig. S1. Schematic quinone redox cycling mechanisms for ROS generation induced by Phanerochaete chrysosporium in the presence of a quinone-type molecule modified by previous study [6].

Fig. S2 (A) The relative abundance of bacterial community at the genus level. (B) The relative abundance of archaeal community at the genus level.

Fig. S3 Abundance of genes related to dissimilatory iron reduction.

Table S1 Comparison of cumulative methane production under the mesophilic condition in the literature to that in this study

| Substrate | Operation mode | Digestion time | Operating parameters | CH4 production (mL/gVS) | Reference |
| --- | --- | --- | --- | --- | --- |
| Cow manure | Batch | 36 days | 37°C | 118.73-148.57 | This study |
| Cow manure | Batch | NR a | 37 ± 1◦C | 49.54-112.38 | [7] |
| Cow manure | Batch | 100 days | 25°C-37°C | 67.5-92.5 | [8] |
| Cow manure | Batch | 36 days | 37°C | 80-105 | [9] |
| Cow manure | Batch | 45 days | NR | 182.46-409.76 | [10] |
| Cow manure and corn straw | Batch | 30 days | 38 ± 1°C | 168.60-230.95 | [11] |
| Dairy manure and swine manure | Batch | 41 days | 37 ± 1°C | 54.3-143.0 | [12] |
| Dairy manure and Cassava Waste | Batch | NR | 35˚C | 80.38-150.33 | [13] |
| Cattle manure and gas silage | Batch | 18 days | 37 ± 1°C | 350 | [14] |
| Cow manure and corn straw | Batch | 30 days | 38 ± 1°C | 65.73-95.93 b | [15] |
| Cow manure | Semi-continuous | SRT=15days | NR | 103-231 | [16] |
| Cow manure and corn straw | Semi-continuous | SRT=25 days | 38℃ | 117 | [17] |
| Cow manure and Grass | Semi-continuous | SRT=65days | 35 ± 1 ◦C | 109.73-331.75 | [18] |

a NR: Not reported.

b mL/g TS

Table S2. Basic information of cow manure.

|  | Hemicellulose, % | Cellulose, % | Lignin, % | Reference |
| --- | --- | --- | --- | --- |
| Cow manure | 26.28 | 29.41 | 14.07 | This study |
| Beef cow manure | 20.00–26.70 | 22.91–42.00 | 8.09–14.00 | [19] |
| Dairy cow manure | 14.05–19.00 | 15.31–29.00 | 13.97–16.00 |  |

Table S3. Alpha diversity of the bacterial community

| Samples | Chao1 | ACE | Shannon | Simpson |
| --- | --- | --- | --- | --- |
| Control group | 5315.01 | 5232.56 | 5.18 | 0.025 |
| IMA group | 5714.63 | 5634.41 | 5.25 | 0.021 |
| IMA-P group | 5431.06 | 5449.34 | 5.30 | 0.021 |

Table S4. Alpha diversity of the archaeal community

| Samples | Chao1 | ACE | Shannon | Simpson |
| --- | --- | --- | --- | --- |
| Control group | 199.00 | 196.11 | 3.05 | 0.09 |
| IMA group | 185.04 | 196.13 | 3.08 | 0.08 |
| IMA-P group | 215.07 | 210.51 | 3.11 | 0.08 |

Reference

[1] X.P. Wang, J.S. Zhang, B.W. Yang, H.H. Mao, Q.L. Yu, Y.B. Zhang, Intermittent Microaeration Enhanced Anaerobic Digestion: The Key Role of Fe(III)/Fe(II) Cycle and Reactive Oxygen Species, Environmental Science & Technology 59(17) (2025) 8629-8639. https://doi.org/10.1021/acs.est.5c04187.

[2] T. Wu, Y. Liu, T. Zheng, Y. Dai, Z. Li, D. Lin, Fe-Based Nanomaterials and Plant Growth Promoting Rhizobacteria Synergistically Degrade Polychlorinated Biphenyls by Producing Extracellular Reactive Oxygen Species, Environ. Sci. Technol. 57(34) (2023) 12771-12781. https://doi.org/10.1021/acs.est.3c02495.

[3] Z. Zhao, Y. Zhang, L. Wang, X. Quan, Potential for direct interspecies electron transfer in an electric-anaerobic system to increase methane production from sludge digestion, Sci Rep 5 (2015) 11094. https://doi.org/10.1038/srep11094.

[4] P.J. Vansoest, J.B. Robertson, B.A. Lewis, Methods for Dietary Fiber, Neutral Detergent Fiber, and Nonstarch Polysaccharides in Relation to Animal Nutrition, Journal of Dairy Science 74(10) (1991) 3583-3597. https://doi.org/10.3168/jds.S0022-0302(91)78551-2.

[5] Y. Gong, Z. Jin, X. Wang, Y. Zhang, Improving methane production and 4-chlorophenol removal in anaerobic digestion of corn straw by adding Phanerochaete chrysosporium and biochar under microaerobic conditions, Water Research 270 (2025). https://doi.org/10.1016/j.watres.2024.122845.

[6] S. Chen, M. Zhu, X. Guo, B. Yang, R. Zhuo, Coupling of Fenton reaction and white rot fungi for the degradation of organic pollutants, Ecotoxicology and Environmental Safety 254 (2023). https://doi.org/10.1016/j.ecoenv.2023.114697.

[7] A. He, Y. Liu, C. Cao, Q. Xiao, J. Xu, Y. Feng, Z. Zhang, Y. Xi, Humic acid–anchored hydrochar for enhancing methane production in anaerobic digestion of cow manure, Journal of Environmental Management 385 (2025). https://doi.org/10.1016/j.jenvman.2025.125703.

[8] M. Silwadi, H. Mousa, B.Y. Al-Hajji, S.S. Al-Wahaibi, Z.Z. Al-Harrasi, Enhancing biogas production by anaerobic digestion of animal manure, International Journal of Green Energy 20(3) (2022) 257-264. https://doi.org/10.1080/15435075.2022.2038608.

[9] Y. Nleya, B. Young, E. Nooraee, S. Baroutian, Anaerobic digestion of dairy cow and goat manure: Comparative assessment of biodegradability and greenhouse gas mitigation, Fuel 381 (2025). https://doi.org/10.1016/j.fuel.2024.133458.

[10] M. Abid, J. Wu, Y. Yuanyuan, Z. Ajmal, T. Mehmood, S.N. Husnain, X. Zhou, Enhanced anaerobic digestion of freezing and thawing pretreated cow manure with increasing solid content: kinetics and microbial community dynamics, Scientific Reports 14(1) (2024). https://doi.org/10.1038/s41598-024-76392-z.

[11] Y. Yang, M. Wang, S. Yan, X. Yong, X. Zhang, M.K. Awasthi, Y. Xi, J. Zhou, Effects of hydrochar and biogas slurry reflux on methane production by mixed anaerobic digestion of cow manure and corn straw, Chemosphere 310 (2023). https://doi.org/10.1016/j.chemosphere.2022.136876.

[12] J. Wi, S. Lee, H. Ahn, Influence of Dairy Manure as Inoculum Source on Anaerobic Digestion of Swine Manure, Bioengineering 10(4) (2023). https://doi.org/10.3390/bioengineering10040432.

[13] R.M.M. Martins, Y.A. da Fonseca, M.V. Faria, S.F. de Aquino, O.F.H. Adarme, B.E. Lobo Baêta, Methane production by anaerobic co‐digestion of dairy manure and cassava wastes for energy recovery, Journal of Chemical Technology & Biotechnology 98(3) (2022) 797-806. https://doi.org/10.1002/jctb.7283.

[14] V. Hidalgo-Sánchez, U. Behmel, J. Hofmann, M.E. Borges, Enhancing Biogas Production of Co-Digested Cattle Manure with Grass Silage from a Local Farm in Landshut, Bavaria, through Chemical and Mechanical Pre-Treatment and Its Impact on Biogas Reactor Hydraulic Retention Time, Sustainability 15(3) (2023). https://doi.org/10.3390/su15032582.

[15] S. Ren, Z. Tong, X. Yong, Y. Xi, F. Liu, J. Zhou, The new strategies of using nitrogen and iron modified hydrochar to enhance methane production during co-anaerobic digestion of cow manure and corn straw, Journal of Environmental Chemical Engineering 12(5) (2024). https://doi.org/10.1016/j.jece.2024.114127.

[16] M. Rasouli, B. Ataeiyan, S.L. Rokhum, Investigation and Optimization of Operational Conditions of Anaerobic Digestion Process for Enhanced Biogas Production Yield in a CSTR Using RSM, International Journal of Energy Research 2024(1) (2024). https://doi.org/10.1155/2024/9158477.

[17] P. Li, Y. Lu, R. Du, H. Meng, Z. Li, C. Liu, H. Cong, X. Yue, B. Ye, Digestate recirculation to enhance biogas production from dry anaerobic digestion of cow manure and corn straw: Strategies and impacts, Biochemical Engineering Journal 222 (2025). https://doi.org/10.1016/j.bej.2025.109805.

[18] A.H. Ulukardesler, Anaerobic co-digestion of grass and cow manure: kinetic and GHG calculations, Scientific Reports 13(1) (2023). https://doi.org/10.1038/s41598-023-33169-0.

[19] Y. Li, J. Zhao, J. Krooneman, G.J.W. Euverink, Strategies to boost anaerobic digestion performance of cow manure: Laboratory achievements and their full-scale application potential, Science of The Total Environment 755 (2021). https://doi.org/10.1016/j.scitotenv.2020.142940.