Supporting Information

Treatment options of nitrogen heterocyclic compounds in industrial wastewater: from fundamental technologies to energy valorization applications and future process design strategies

Chao Maa,1, Huiqin Zhanga,1, Ziwei Liub*, Xinran Mengb, Sijia Chenb, Jingsong Zhangd, Yeqiang Lid, Xia Huangb,c*

a Innovation Demonstration Base of Ecological Environment Geotechnical and Ecological Restoration of Rivers and Lakes, Hubei University of Technology, Wuhan,430068, P.R. China

b State Key Joint Laboratory of Environment Simulation and Pollution Control, School of Environment, Tsinghua University, Beijing, 100084, P.R. China

c Research and Application Center for Membrane Technology, School of Environment, Tsinghua University, Beijing, 100084, P.R. China

d CITIC Envirotech Pte. (Guangzhou) Ltd., Guangzhou, 511455, P.R. China

1These authors contributed equally.

*Corresponding authors. E-mail address: ziweiliu@mail.tsinghua.edu.cn (Ziwei Liu), xhuang@tsinghua.edu.cn (Xia Huang)

Contents:

Table S1. Industrial source of NHCs                                    P 1

Table S2. The operational conditions and adsorption performances of commonly used adsorbents for NHCs                                                 P 2

Table S3. The operational conditions and NHC degradation performances of advanced oxidation processes                                                  P 4

Table S4. Enhanced microbial treatment of NHCs with the addition of functional bacteria                                                            P 6

Table S5. Enhanced microbial treatment of NHCs with the addition of enzyme   P 8

Table S6. The processes treating NHC-containing real industrial wastewater     P 9

Table S1. Industrial source of NHCs

| Industry | Wastewater | Types of NHCs | Concentration (mg/L) | Reference |
| --- | --- | --- | --- | --- |
| Pharmaceutical industry | synthetic wastewater | Quinazoline Metronidazole | 5000 200 | (Gong et al., 2016) (Xin et al., 2024) |
| Coal processing | coal gasification wastewater synthetic wastewater | Pyridine and derivatives Indole Quinoline | 25~400 25~600 25~600 | (Li et al., 2019; Shi et al., 2019a; Shi et al., 2019b; Shi et al., 2019c; Gao et al., 2020; Shi et al., 2020) |
| Refine oil | synthetic wastewater | Pyridine and derivatives Pyrrole and derivatives Indole Quinoline | 50~1000 1000 100~2000 100~2000 | (Ahmed and Jhung, 2016; Shin et al., 2019; Khan et al., 2020; Sun et al., 2022) |
| Chemical Industries | hydrothermal liquefaction aqueous phase synthetic wastewater | Pyridine and derivatives Pyrazine and derivatives Pyrrole and derivatives Indole and derivatives Quinoline | 500~4000 300~5600 300 10~100 50~130 | (Ben Hammouda et al., 2016; Wang et al., 2016; Zhou et al., 2017; Huang et al., 2020; Ciarlini et al., 2024; Yang et al., 2024) |
| Herbicide | synthetic wastewater | Atrazine | 20~30 | (Tian et al., 2017; Jia et al., 2024) |

Table S2. The operational conditions and adsorption performances of commonly used adsorbents for NHCs

| Adsorbent | Wastewater | Types of NHCs | Concentration (mg/L) | HRT (h) | pH | Temperature(℃) | Optimal conditions | Kinetic model | qe,max (mg/g) | Reference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Granular activated carbon (GAC) | synthetic wastewater | Pyridine | 20~800 | 72 | 4~12 | 15, 25, 35 | pH=10, 25℃ | Prausnitz-Radke | 129.4 | (Ocampo-Perez et al., 2010) |
| Granular activated carbon (GAC) | synthetic wastewater | Quinoline | 50~1000 | 4~12 | 2~12 | 15~45 | pH=5.5, 45℃, 24h | Langmuir | 174.72 | (Rameshraja et al., 2012) |
| Bamboo-based activated carbon | synthetic wastewater | Pyridine, indole and quinoline | 5~200 | 24 | 7±0.2 | 25, 35, 45 | pH=7.0, 25℃ | Pseudo-second-order | 15.8, 19.5, 17.7 | (Liao et al., 2013) |
| Modified activated carbon | synthetic wastewater | Quinoline and indole | 300~1200 | 2~6 | / | 25 | 25℃, 6h | Langmuir | 126, 168 | (Ahmed and Jhung, 2015) |
|  |  | Quinoline | 500~10000 | 1~24 | / | 30 | 30℃, 24h | Langmuir | 56.6 | (de Oliveira Ferreira et al., 2019) |
| MOF (GnO(1.0)/MIL-101) | synthetic wastewater | Quinoline and indole | 250~2000 | / | / | 25 | / | Langmuir | 498, 542 | (Ahmed and Jhung, 2016) |
| MOF (GnO(3.0)/MIL-101) | synthetic wastewater | Quinoline and indole | 250~2000 | / | / | 25 | / | Langmuir | 484, 593 | (Ahmed and Jhung, 2016) |
| MOF (P-Ade(1)-MIL-101) | synthetic wastewater | Quinoline and indole | 500~2000 | 6~12 | / | 20 | 20℃, 12h | Langmuir | 511, 532 | (Sarker et al., 2018) |
| MOF (MIL-125-VFG) | synthetic wastewater | Quinoline and indole | 500~2000 | 3~6 | / | 25 | 25℃, 6h | Langmuir | 546, 637 | (Shin et al., 2019) |
| MOF (Cu2O(10)/MDC-K) | synthetic wastewater | Quinoline and indole | 100~1500 | 12 | / | 25 | / | Langmuir | 454, 435 | (Khan et al., 2020) |
| Hypercrosslinked Resins (MN 500) | synthetic wastewater | Pyridine | 50~400 | 2~24 | 4.3~10.3 | 25, 35, 45, 55 | pH=7.3, 25℃ | Langmuir | 86 | (Zhu et al., 2016) |
| Hypercrosslinked Resins (70% sulfonatete) | synthetic wastewater | Pyridine | 100~1000 | 1.5~24 | 1~11 | 25, 35, 45 | pH=8, 25℃ | Langmuir | 210 | (Sun et al., 2022) |

Table S3. The operational conditions and NHC degradation performances of advanced oxidation processes

| AOP methods | Wastewater | Types of NHCs | Concentration (mg/L) | Reaction time (min) | pH | Temperature(℃) | Treatment effect | Reference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Fenton oxidation | synthetic wastewater | Indole | 20 | 180 | 3.0 | Room | The removal rates of indole and TOC were 100% and 72% respectively | (Ben Hammouda et al., 2016) |
|  | synthetic wastewater | 3-methylindole | 20 | 120 | 3.0 | Room | The removal rates of 3-methylindole and TOC were 100% and 80% respectively | (Ben Hammouda et al., 2015) |
| Fenton-like oxidation (CuCe oxide +H2O2) | synthetic wastewater | Quinoline | 100 | 85 | 3.8~10.5 | Room | The removal rates of quinoline and TOC were 98.1% and 86.1% respectively | (Jiao et al., 2022) |
| Photocatalytic oxidation | synthetic wastewater | Quinoline | 71 | 180 | 6.06 | Room | The removal rates of quinoline was 91.5% | (Jing et al., 2012) |
|  | synthetic wastewater | Quinoline | 50 | 240 | 8.0 | Room | The removal rates of quinoline and TOC were 92% and 78% respectively | (Gupta et al., 2020) |
| Electrochemical oxidation | synthetic wastewater | Pyridine | 100 | 180 | 3.0 | Room | The removal rates of pyridine and TOC were 98% and 86% respectively | (Li et al., 2016) |
|  | synthetic wastewater | Pyridine | 200 | 360 | 3.0 | Room | The removal rates of pyridine was 82.9% | (Wang et al., 2015) |
| Catalytic ozone oxidation | synthetic wastewater | Quinoline | 20 | 15 | 6.8 | Room | The removal rates of quinoline and TOC were 100% and 31.5% respectively | (Zhu et al., 2017b) |
|  | synthetic wastewater | Quinoline | 50 | 30 | 6.8 | Room | The removal rates of quinoline and TOC were 100% and 90.3% respectively | (Liu et al., 2019) |
| Supercritical water oxidation | synthetic wastewater | Quinazoline | 5000 | 7 | / | 600 | The removal rates of quinazoline, TOC and TN were 100%, 97.2% and 70.7% respectively | (Gong et al., 2016) |
|  | synthetic wastewater | Indole | 120 | 8 | / | 450 | The removal rates of indole, TOC and TN were 100%, 69.7 % and 44.7 % respectively | (Yang et al., 2022) |

Table S4. Enhanced microbial treatment of NHCs with the addition of functional bacteria

| Microorganisms | Substrate | Types of NHCs | Findings | Reference |
| --- | --- | --- | --- | --- |
| Stenotrophomonas maltophilia J2 | Medium & synthetic wastewater | Pyridine | 1100mg/L pyridine: 98.34% degradation of within 72 hours. 2000mg/L pyridine: 98.66% degradation of immobilised bacteria within 24 hours. | (Niu et al., 2023) |
| Paracoccus sp. NJUST30 | Medium | Pyridine | 500mg/L pyridine: pyridine completely (and 91.83% of the total organic carbon) degradation of within 54 hours. | (Wang et al., 2018) |
| Rhodococcus Chr-9 | Medium | Pyridine | 200mg/L pyridine and 200mg/L phenol: degradation of within 30 hours. 3-Hydroxypyridine and quinoline can also be utilised for degradation. | (Sun et al., 2011) |
| Exiguobacterium aurantiacum (MTCC 5638) | Medium & synthetic wastewater | Pyridine and quinoline | The organic loading rate of 6.7-16.1 g-COD/m2/d and the removal efficiency of organics (ten organics such as pyridines, quinolines, phenols, and polycyclic aromatic hydrocarbons) was maintained at 91-92%. | (Jeswani and Mukherji, 2015) |
| Bacillus tropicus, Bacillus aquimaris and Rhodococcus ruber | Medium & synthetic wastewater | Pyridine, quinoline, 2-hydroxypyridine and 2-hydroxyquinoline | 79mg/L pyridine: pyridine completely (and 87% of the total organic carbon) degradation of within 20 hours. 129mg/L quinoline: quinoline completely (and 98% of the total organic carbon) degradation of within 6 hours. By combining Rhodococcus ruber with any of the other two functional bacteria, the removal efficiencies of pyridine and quinoline are enhanced (7-23% and 6-16%). | (Zhu et al., 2021a; Zhu et al., 2021b) |
| Bacillus sp. Q2 | Medium & real wastewater | Quinoline | 500 mg/L quinoline: degradation of within 30 hours. When this strain is added to a pilot-scale A-O system, it can significantly reduce the COD of the effluent and achieve complete degradation of quinoline. | (Tuo et al., 2012) |
| Comamonas testosterone and Rhodococcus ruber | Medium & synthetic wastewater | Quinoline and 2-hydroxyquinoline | 32.25mg/L quinoline: quinoline completely (and 79% of the total organic carbon) degradation of within 2 hours. Nitrification rates of quinoline were increased by 6.3 and 4.3 times with the addition of Comamonas testosterone and Rhodococcus ruber alone. | (Zhu et al., 2023) |
| Phomopsis liquidambari | Medium | Indole | 100mg/L indole: 99.1% degradation of within 60 hours. Discovery that laccase and LiP can break the metabolic bottleneck of the intermediate products oxindole and isatin | (Chen et al., 2013) |
| Burkholderia sp. IDO3 | Medium | Indole | 100mg/L indole: degradation of within 14 hours. discovery of a new indole dioxygenase: IifC_IDO3. | (Ma et al., 2019) |
| Cupriavidus sp. KK10 | Medium | Indole | 50mg/L indole: degradation of within 24 hours. | (Fukuoka et al., 2015) |

Table S5. Enhanced microbial treatment of NHCs with the addition of enzyme

| Enzyme | Substrate | Types of NHCs | Findings | Reference |
| --- | --- | --- | --- | --- |
| Laccase-ABST/ Calcium alginate microspheres | Medium & synthetic wastewater | Acridine | 30 mg/L acridine: degradation of within 8 h | (Xue et al., 2020) |
| Laccase-ABST/ Calcium alginate microspheres | Medium & synthetic wastewater | Indole | 15 mg/L indole: degradation rate 99.7% within 24 h; Catalyst: maintains high catalytic activity after being recycled 10 times or stored for 100 days | (Gu et al., 2019) |
| Laccase-HBT/ Bacterial cellulose | Medium & synthetic & real wastewater | Atrazine | 20 mg/L Atrazine: Atrazine completely (and 85% of the intermediate products) degraded within 5 h | (Jia et al., 2024) |
| Horseradish Peroxidase-SY/ Calcium alginate microspheres | Medium & synthetic wastewater | Indole | 25~500 mg/L indole: completely degraded under optimal conditions in the presence of H2O2 (100 mg/L indole: completely degraded within 40 h; 500 mg/L indole: completely degraded within 60 h) | (Liu et al., 2021) |

Table S6. The technologies/processes treating NHC-containing real industrial wastewater

| Methods | Wastewater | Types of NHCs | Concentration (mg/L) | COD (BOD5) (mg/L) | TOC (mg/L) | Nitrogen (mg/L) | Degradation efficiency | Other information | Reference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Electro-Fenton | Pharmaceutical wastewater | sulfadiazine, sulfadimethoxine, sulfathiazole | 5.34, 4.94, 5.48 | 1300 (190) | 621 | / | 64.17% of TOC removal within 3 h | Conductivity 6.4 mS/cm; pH 6.88 | (Zhu et al. 2022) |
| Electrochemical oxidation-PMS | Synthetic wastewater, Biologically pretreated coal gasification wastewater | Pyridine (synthetic), Unknown (real) | 50 (synthetic) | 590±20 (real) | 170±5 (real) | Ammonia-N 0.13 (real) | 93.5% of Pyridine removal within 1 h (synthetic) | Conductivity 6.95 mS/cm; pH 7.62±0.05 (real) | (He et al. 2025) |
| AGS with iron-carbon | Textile-dyeing wastewater | quinoline | 1000 | 800 | / | Total-N 60-70 | 80% of COD removal, 90% of quinoline removal (batch, HRT=12.5 h) | / | (Pan et al. 2024) |
| Anoxic-aerobic vertical baffled bioreactor (pilot-scale) | N-Methyl pyrrolidone manufacturing wastewater | N-Methyl pyrrolidone (NMP), methylamine | 1584, 465 | 3042±50 | / | Ammonia-N 53.6 NMP-N 224 Methylamine-N 210 | Complete removal of ammonium, ~50% of total-N removal within 2.5 h | pH 10±0.5; re-circulation ratio of 200% | (Chen et al. 2024) |
| Fenton oxidation-aerobic (P. pseudoalcaligenes) process | Chemical industry wastewater | 3-cyanopyridine | 725 | 25624 (3125) | / | Ammonia-N 14175 | 99% of 3-cyanopyridine, 93% of TOC removal within 3 h Fenton+288 h biodegradation | pH 9.43 | (Padoley et al. 2011) |
|  |  | pyridine | 1700 | 65000 (24160) | / | Ammonia-N 37632 | 84% of pyridine, 82% of TOC removal within 3 h Fenton+288 h biodegradation | pH 9.95 |  |
| Catalytic ozone-anoxic-aerobic MBR system | Biologically pretreated coal gasification wastewater | quinoline, pyridine, indole | 20, 20, 10 | 150-180 (~10) | 60-70 | Ammonia-N 20-30 Total-N 50-60 | Complete removal of quinoline, pyridine, and indole; 92.7%, 95.6%, and 80.6% of COD, ammonia-N, and total-N removal within 0.5 h catalytic ozonation+12 h biodegradation | pH 7.5-8.5 | (Zhu et al. 2017a) |

Reference

Ahmed I, Jhung S H (2015). Remarkable improvement in adsorptive denitrogenation of model fossil fuels with CuCl/activated carbon, prepared under ambient condition. Chemical Engineering Journal, 279: 327-334

Ahmed I, Jhung S H (2016). Remarkable adsorptive removal of nitrogen-containing compounds from a model fuel by a graphene oxide/MIL-101 composite through a combined effect of improved porosity and hydrogen bonding. Journal of Hazardous materials, 314: 318-325

Ben Hammouda S, Adhoum N, Monser L (2015). Synthesis of magnetic alginate beads based on Fe3O4 nanoparticles for the removal of 3-methylindole from aqueous solution using Fenton process. Journal of Hazardous materials, 294: 128-136

Ben Hammouda S, Adhoum N, Monser L (2016). Chemical oxidation of a malodorous compound, indole, using iron entrapped in calcium alginate beads. Journal of Hazardous materials, 301: 350-361

Chen, S., Zhu, X., Zhu, G., Liang, B., Luo, J., Zhu, D., Chen, L., Zhang, Y. and Rittmann, B.E. (2024) N-methyl pyrrolidone manufacturing wastewater as the electron donor for denitrification: From bench to pilot scale. Sci. Total Environ. 912, 169517.

Chen Y, Xie X-G, Ren C-G, Dai C-C (2013). Degradation of N-heterocyclic indole by a novel endophytic fungus Phomopsis liquidambari. Bioresource Technology, 129: 568-574

Ciarlini J, Haynes B S, Montoya A (2024). Carbon and nitrogen product distribution in the electrochemical degradation of nitrogen-heterocyclic compounds. Chemical Engineering Journal, 480: 148110

De Oliveira Ferreira M E, Vaz B G, Borba C E, Alonso C G, Ostroski I C (2019). Modified activated carbon as a promising adsorbent for quinoline removal. Microporous and Mesoporous Materials, 277: 208-216

Fukuoka K, Tanaka K, Ozeki Y, Kanaly R A (2015). Biotransformation of indole by Cupriavidus sp. strain KK10 proceeds through N-heterocyclic- and carbocyclic-aromatic ring cleavage and production of indigoids. International Biodeterioration & Biodegradation, 97: 13-24

Gao Y, Kong X, Zhou A, Yue X, Luo Y, Defemur Z (2020). Enhanced degradation of quinoline by coupling microbial electrolysis cell with anaerobic digestion simultaneous. Bioresource Technology, 306: 123077

Gong Y, Guo Y, Wang S, Song W (2016). Supercritical water oxidation of Quinazoline: Effects of conversion parameters and reaction mechanism. Water Research, 100: 116-125

Gu Y, Xue P, Jia F, Shi K (2019). Co-immobilization of laccase and ABTS onto novel dual-functionalized cellulose beads for highly improved biodegradation of indole. Journal of Hazardous materials, 365: 118-124

Gupta D, Chauhan R, Kumar N, Singh V, Srivastava V C, Mohanty P, Mandal T K (2020). Enhancing photocatalytic degradation of quinoline by ZnO:TiO2 mixed oxide: Optimization of operating parameters and mechanistic study. Journal of Environmental Management, 258: 110032

He, L., Carrera, P., Wang, C., Duc, V. N., Heynderickx, P. M., Feng, R. F., Guo, S. and Wu, D. (2025) Assessment of hydrogen peroxide, persulfate, and peroxymonosulfate as oxidizing agents in electrochemical oxidation of pyridine. Chem. Eng. J. 503, 158125.

Huang L, Liu Z, Dong H, Yu T, Jiang H, Peng Y E, Shi L (2020). Coupling quinoline degradation with Fe redox in clay minerals: A strategy integrating biological and physicochemical processes. Applied Clay Science, 188: 105504

Jeswani H, Mukherji S (2015). Treatment of simulated biomass gasification wastewater of varying strength in a three stage rotating biological contactor. Chemical Engineering Journal, 259: 303-312

Jia J, Xue P, Ma L, Li P, Xu C (2024). Deep degradation of atrazine in water using co-immobilized laccase-1-hydroxybenzotriazole-Pd as composite biocatalyst. Journal of Hazardous materials, 468: 133779

Jiao Z, Zhang X, Gong H, He D, Yin H, Liu Y, Gao X (2022). CuO-doped Ce for catalytic wet peroxide oxidation degradation of quinoline wastewater under wide pH conditions. Journal of Industrial and Engineering Chemistry, 105: 49-57

Jing J, Li W, Boyd A, Zhang Y, Colvin V L, Yu W W (2012). Photocatalytic degradation of quinoline in aqueous TiO2 suspension. Journal of Hazardous materials, 237-238: 247-255

Khan N A, Shin S, Hwa Jhung S (2020). Cu2O-incorporated MAF-6-derived highly porous carbons for the adsorptive denitrogenation of liquid fuel. Chemical Engineering Journal, 381: 122675

Li D, Tang J, Zhou X, Li J, Sun X, Shen J, Wang L, Han W (2016). Electrochemical degradation of pyridine by Ti/SnO2–Sb tubular porous electrode. Chemosphere, 149: 49-56

Li K, Shi J, Han Y, Xu C, Han H (2019). Enhanced anaerobic degradation of quinoline, pyriding, and indole with polyurethane (PU), Fe3O4@PU, powdered activated carbon (PAC), Fe(OH)3@PAC, biochar, and Fe(OH)3@biochar and analysis of microbial succession in different reactors. Bioresource Technology, 291: 121866

Liao P, Yuan S, Xie W, Zhang W, Tong M, Wang K (2013). Adsorption of nitrogen-heterocyclic compounds on bamboo charcoal: Kinetics, thermodynamics, and microwave regeneration. Journal of Colloid and Interface Science, 390(1): 189-195

Liu D, Wang C, Song Y, Wei Y, He L, Lan B, He X, Wang J (2019). Effective mineralization of quinoline and bio-treated coking wastewater by catalytic ozonation using CuFe2O4/Sepiolite catalyst: Efficiency and mechanism. Chemosphere, 227: 647-656

Liu X, Xue P, Jia F, Shi K, Gu Y, Ma L, Li R (2021). A novel approach to efficient degradation of indole using co-immobilized horseradish peroxidase-syringaldehyde as biocatalyst. Chemosphere, 262: 128411

Ma Q, Liu Z, Yang B, Dai C, Qu Y (2019). Characterization and functional gene analysis of a newly isolated indole-degrading bacterium Burkholderia sp. IDO3. Journal of Hazardous materials, 367: 144-151

Niu H, Nie Z, Long Y, Guo J, Tan J, Bi J, Yang H (2023). Efficient pyridine biodegradation by Stenotrophomonas maltophilia J2: Degradation performance, mechanism, and immobilized application for wastewater. Journal of Hazardous materials, 459: 132220

Ocampo-Perez R, Leyva-Ramos R, Alonso-Davila P, Rivera-Utrilla J, Sanchez-Polo M (2010). Modeling adsorption rate of pyridine onto granular activated carbon. Chemical Engineering Journal, 165(1): 133-141

Padoley, K.V., Mudliar, S.N. and Pandey, R.A. (2008) Heterocyclic nitrogenous pollutants in the environment and their treatment options – An overview. Bioresour. Technol. 99(10), 4029-4043.

Pan, K., Qian, Z., Chen, Y., Cheng, X., Li, F., Ding, M., Tian, Y. and Li, J. (2024) Two-way role of iron-carbon in biochemical reactions: Microelectrolysis and enhanced activity of aerobic granular sludge for efficient refractory wastewater treatment. Water Res. 264, 122222.

Rameshraja D, Srivastava V C, Kushwaha J P, Mall I D (2012). Quinoline adsorption onto granular activated carbon and bagasse fly ash. Chemical Engineering Journal, 181-182: 343-351

Sarker M, Song J Y, Jeong A R, Min K S, Jhung S H (2018). Adsorptive removal of indole and quinoline from model fuel using adenine-grafted metal-organic frameworks. Journal of Hazardous materials, 344: 593-601

Shi J, Han H, Xu C (2019a). A novel enhanced anaerobic biodegradation method using biochar and Fe(OH)3@biochar for the removal of nitrogen heterocyclic compounds from coal gasification wastewater. Science of the Total Environment, 697: 134052

Shi J, Han Y, Xu C, Han H (2019b). Enhanced anaerobic degradation of selected nitrogen heterocyclic compounds with the assistance of carboxymethyl cellulose. Science of the Total Environment, 689: 781-788

Shi J, Xu C, Han Y, Han H (2019c). Enhanced anaerobic biodegradation efficiency and mechanism of quinoline, pyridine, and indole in coal gasification wastewater. Chemical Engineering Journal, 361: 1019-1029

Shi J, Xu C, Han Y, Han H (2020). Enhanced anaerobic degradation of nitrogen heterocyclic compounds with methanol, sodium citrate, chlorella, spirulina, and carboxymethylcellulose as co-metabolic substances. Journal of Hazardous materials, 384: 121496

Shin S, Sarker M, Lee H-I, Jhung S H (2019). Metal-organic framework with various functional groups: Remarkable adsorbent for removal of both neutral indole and basic quinoline from liquid fuel. Chemical Engineering Journal, 370: 1467-1473

Sun J-Q, Xu L, Tang Y-Q, Chen F-M, Liu W-Q, Wu X-L (2011). Degradation of pyridine by one Rhodococcus strain in the presence of chromium (VI) or phenol. Journal of Hazardous materials, 191(1): 62-68

Sun Y, Gu Y, Yang J (2022). Adsorption of N-heterocyclic compounds from aqueous solutions by sulfonic acid-functionalized hypercrosslinked resins in batch experiments. Chemical Engineering Journal, 428: 131163

Tian Y, Shen W, Jia F, Ai Z, Zhang L (2017). Sulfite promoted photochemical cleavage of s-triazine ring: The case study of atrazine. Chemical Engineering Journal, 330: 1075-1081

Tuo B-H, Yan J-B, Fan B-A, Yang Z-H, Liu J-Z (2012). Biodegradation characteristics and bioaugmentation potential of a novel quinoline-degrading strain of Bacillus sp. isolated from petroleum-contaminated soil. Bioresource Technology, 107: 55-60

Wang C, Ma K, Wu T, Ye M, Tan P, Yan K (2016). Electrochemical mineralization pathway of quinoline by boron-doped diamond anodes. Chemosphere, 149: 219-223

Wang J, Jiang X, Liu X, Sun X, Han W, Li J, Wang L, Shen J (2018). Microbial degradation mechanism of pyridine by Paracoccus sp. NJUST30 newly isolated from aerobic granules. Chemical Engineering Journal, 344: 86-94

Wang Z, Zhang B, Borthwick A G L, Feng C, Ni J (2015). Utilization of single-chamber microbial fuel cells as renewable power sources for electrochemical degradation of nitrogen-containing organic compounds. Chemical Engineering Journal, 280: 99-105

Xin H, Chen X, Ye Y, Liao Y, Luo H, Tang C Y, Liu G (2024). Enhanced metronidazole removal in seawater using a single-chamber bioelectrochemical system. Water Research, 252: 121212

Xue P, Liu X, Gu Y, Zhang W, Ma L, Li R (2020). Laccase-mediator system assembling co-immobilized onto functionalized calcium alginate beads and its high-efficiency catalytic degradation for acridine. Colloids and Surfaces B: Biointerfaces, 196: 111348

Yang B, Ma Q, Ren X, Peng X, Wang H, Li L, Hao J (2022). Supercritical Water Oxidation of Aniline, Nitrobenzene, and Indole: Effect of Catalysts on Nitrogen Conversion Mechanism. The Journal of Supercritical Fluids, 188: 105680

Yang J, Liu K, Yi W, Si B, Tian C, Yang G (2024). Effects of biochar, granular activated carbon, and magnetite on the electron transfer of microbials during the anaerobic digestion process: Insights into nitrogen heterocyclic compounds degradation. Fuel, 358: 130079

Zhou X, Liu S, Xu A, Wei K, Han W, Li J, Sun X, Shen J, Liu X, Wang L (2017). A multi-walled carbon nanotube electrode based on porous Graphite-RuO2 in electrochemical filter for pyrrole degradation. Chemical Engineering Journal, 330: 956-964

Zhu G, Xing F, Tao J, Chen S, Li K, Cao L, Yan N, Zhang Y, Rittmann B E (2021a). Synergy of strains that accelerate biodegradation of pyridine and quinoline. Journal of Environmental Management, 285: 112119

Zhu G, Zhang H, Yuan R, Huang M, Liu F, Li M, Zhang Y, Rittmann B E (2023). How Comamonas testosteroni and Rhodococcus ruber enhance nitrification in the presence of quinoline. Water Research, 229: 119455

Zhu G, Zhang Y, Chen S, Wang L, Zhang Z, Rittmann B E (2021b). How bioaugmentation with Comamonas testosteroni accelerates pyridine mono-oxygenation and mineralization. Environmental Research, 193: 110553

Zhu, H., Han, Y., Ma, W., Han, H. and Ma, W. (2017a) Removal of selected nitrogenous heterocyclic compounds in biologically pretreated coal gasification wastewater (BPCGW) using the catalytic ozonation process combined with the two-stage membrane bioreactor (MBR). Bioresour. Technol. 245, 786-793.

Zhu H, Ma W, Han H, Han Y, Ma W (2017b). Catalytic ozonation of quinoline using nano-MgO: Efficacy, pathways, mechanisms and its application to real biologically pretreated coal gasification wastewater. Chemical Engineering Journal, 327: 91-99

Zhu Q, Moggridge G D, Ainte M, Mantle M D, Gladden L F, D’agostino C (2016). Adsorption of pyridine from aqueous solutions by polymeric adsorbents MN 200 and MN 500. Part 1: Adsorption performance and PFG-NMR studies. Chemical Engineering Journal, 306: 67-76

Zhu, Y., Deng, F., Qiu, S., Ma, F., Zheng, Y. and Gao, L. (2022b) A self-sufficient electro-Fenton system with enhanced oxygen transfer for decontamination of pharmaceutical wastewater. Chem. Eng. J. 429, 132176.