Supporting Information

Intermittent air mixing system for anaerobic digestion of animal wastewater: Operating conditions and full-scale validation

Hongnan Yanga,b, Liangwei Denga,b*, Jianwang Wuc, Wenguo Wanga,b, Dan Zhenga,b, Zhiyong Wanga,b, Yi Liua,b

aBiogas Institute of Ministry of Agriculture and Rural Affairs, Chengdu 610041, P. R. China

bKey Laboratory of Development and Application of Rural Renewable Energy, Ministry of Agriculture and Rural Affairs, Chengdu 610041, P. R. China

cShijiazhuang Chengding Environmental Protection Technology Co. Ltd., Shijiazhuang 050000, P. R. China

*Corresponding author:

E-mail: dengliangwei@caas.cn

Tel/ Fax: +86-28-85236376

Table：S1-3

Figure：S1-2

Supporting tables

Table S1 Characteristics of swine wastewater and sludge inoculum

| Parameter | Swine wastewater | Sludge inoculum |
| --- | --- | --- |
| Total solid (TS) (g/L) | 60.33 ±1.93 | 82.46 ±0.15 |
| Volatile solid (VS) (g/L) | 47.38 ±1.77 | 48.73 ±0.52 |
| Suspended solid (SS) (g/L) | 51.29 ±12.04 |  |
| TCOD (g/L) | 59.79 ±5.85 |  |
| SCOD (g/L) | 6.75 ±1.23 |  |
| TCOD/SCOD | 8.54 ±1.42 |  |
| pH | 7.1 ±0.3 | 6.8 ±0.2 |
| ORP (mV) | -312.7 ±9.9 |  |

Table S2 Overview of the facilities and equipment used at the biogas plant for treating swine wastewater

| Item | Volume | Specification of affiliated equipment |
| --- | --- | --- |
| Collecting tank | 500 m3 | Lift pump: Flow rate 100 m3/h, head 20 m, power 7.5 kw |
| Vertical sedimentation tank | 300 m3 | CSTR feeding pump: Flow rate 100 m3/h, head 20 m, power 7.5 kw; UASB feeding pump: Flow rate 20 m3/h, head 30 m, Power 3 kw |
| CSTR for the anaerobic digestion of the concentrated liquid | 2,000 m³ |  |
| UASB for the anaerobic digestion of low content liquid | 450 m³ |  |
| Air buffer tank | 1.0 m³ | Air compressor: Pressure 0.8 MPa, Flow rate 0.6 m3/h, Power 4 kw |
| Biogas holder | 500 m3 | Air blower: Flow rate 0.6 500 m3/h，Pressure 0.02 MPa， Power 4 kw |
| Electric generator | 500 kW | Power 200 kw |

CSTR: continuous stirred-tank reactor; UASB: upflow anaerobic sludge blanket.

Table S3 Effects of the mixing intensity on the biogas production time, biogas composition, and degree of mixing

| Mixing intensity (Air flow rate) | 33.3 L/L/min | 66.7 L/L/min |
| --- | --- | --- |
| Volumetric biogas production rate (L/L/min) | 2.377 ±0.198 | 2.543 ±0.130 |
| CH4 content (%) | 53.0 ±2.2 | 53.8 ±1.5 |
| O2 content (%) | 3.7 ±0.4 | 2.2±0.2 |
| N2 content (%) | 13.5 ±0.5 | 8.3 ±0.7 |
| H2S (ppm) | 88 ±48 | 100 ±65 |

Supporting Figures

Fig. S1. Schematic diagram showing components of air mixed reactor.

Fig. S2. Flow diagram illustrating the operation of a full-scale biogas plant.

Figure S3. Influence of the air mixing time on the oxidation-reduction potential (ORP).