Supporting Information

Anaerobic Digestion of Dairy Dissolved Air Flotation Sludge: Assessment of Enhancement Strategies

Iin Parlinaa, Asaf Rachmanib, Saeid Baroutiana,c,*

aDepartment of Chemical and Material Engineering, The University of Auckland, 5 Grafton Road, Auckland, 1010, New Zealand

bFonterra Co-operative Group Ltd., 109 Fanshawe Street, Auckland, 1010, New Zealand

cCircular Innovations (CIRCUIT) Research Centre, The University of Auckland, 5 Grafton Road, Auckland, 1010, New Zealand

Table S1. The criteria and sub-criterion, the weight used in this study

| Criteria | Sub-Criteria | Description | Weight for Pretreatment and Process Modification Technology (%) | Weight for Additives and Co-Substrate for Anaerobic co-digestion Technology (%) |
| --- | --- | --- | --- | --- |
| Technical Aspects (40%) | AD enhancement | - Percentage of biogas or methane increased after the technology application | 10 | 20 |
|  | How the tech solutions address the challenges | - How well the technological options can address the key challenge | 20 | 20 |
|  | Technology Readiness Level | - Technology maturity and readiness levels for deployment range from early-stage research and development to fully operational systems. | 10 | - |
| Operational Aspects (20%) | Maintenance Requirements | - Interventions, such as cleaning, inspection, and replacement of parts, which can reduce labour costs and minimize downtime | 5 | 4 |
|  | Operational complexity and ease of operation | - How easy the operation can be applied without intensive training requirement and complicated steps to do the operation. | 5 | 4 |
|  | Integration with existing AD system | - Minimise modification to the current process, reduce the need for additional equipment, and lower capital investment costs | 5 | 4 |
|  | Health and safety requirements | - Risks of possible accidents or environmental hazards when handling chemicals and operating equipment and less generation of toxic by-products | 5 | 4 |
|  | Handling and storage | - The practical aspects of storing and managing the substrate. It deals with space requirements, handling equipment, preservation methods, and potential losses during storage. | - | 4 |
| Economical Aspects (20%) | Capital cost | - Number of financial resources required to implement the technology (purchase of land, buildings, construction, and equipment) | 10 | 10 |
|  | Operational cost | - Expenses which are related to the continuous operation of the technology (E.g.: energy costs such as cost of power for operations and cost of salaries/wages of specialist operators if required) | 10 | 10 |
| Environmental Aspects (20%) | Waste generation | - The amount and type of waste generated (including the level of hazard of the chemical handled), also its potential environmental and economic implications. The more hazardous the chemical is, more complex treatment needed before discharging to the environment | 10 | 10 |
|  | Gas emission | - The amount and type of gaseous emissions produced during the operation of the technology, including greenhouse gases (GHGs) like CO2, CH4, and N2O, as well as other pollutants like NOX, SOX, and volatile organic compounds (VOCs). | 10 | 10 |

Table S2. Scoring guidelines for the technical aspect of biogas production to assess technological solutions

| Biogas Production |  |
| --- | --- |
| How much is the improvement after the technology application |  |
| Score | Scoring guidelines |
| 1 | 0-20% |
| 2 | 21-40% |
| 3 | 41%-60% |
| 4 | 61%-80% |
| 5 | 81%-100% |
| 6 | 101%-150% |
| 7 | 151%-200% |
| 8 | 201%-250% |
| 9 | 251%-300% |
| 10 | 301%-350% |
| 11 | 351%-400% |
| 12 | 401%-450% |
| 13 | 451%-500% |
| 14 | 501%-500% |
| 15 | 551%-500% |
| 16 | 601%-500% |
| 17 | 651%-500% |
| 18 | 701%-500% |
| 19 | 751%-500% |
| 20 | 801%-500% |
| 21 | 851%-500% |
| 22 | 901%-500% |
| 23 | 951%-1000% |
| 24 | 1001%-1500% |
| 25 | 1501%-2000% |

Table S 3. Scoring guidelines for the technical aspect of Technology Readiness Level to assess technological solutions under pretreatment and process modification categories

| Technology Readiness Level |  |
| --- | --- |
| the maturity and readiness of a technology for deployment. It ranges from early-stage research and development to fully operational systems. |  |
| Score | Scoring guidelines |
| 1 | TRL 1 - Basic principles observed: the technology is at the initial stage of research. basic principles are being studied, but there is no experimental proof of concept yet. high uncertainty and risk are associated with this stage. |
| 2 | TRL 2 - Technology concept formulated: The concept and application of the technology have been formulated. Basic studies are conducted to understand the feasibility, but no experimental validation has been done. |
| 3 | TRL 3 - Experimental proof of concept: Active research and development are underway. Laboratory studies are conducted to prove the technical feasibility of the technology. |
| 4 | TRL 4 - Technology validated in lab: The technology is validated in a laboratory environment. Key components and processes are integrated to demonstrate the performance under controlled conditions. |
| 5 | TRL 5 - Technology validated in relevant environment: The technology is tested and validated in a relevant environment, simulating real-world conditions. This stage bridges the gap between laboratory validation and field testing. |
| 6 | TRL 6 - Technology demonstrated in relevant environment: A prototype system is demonstrated in a relevant environment. The technology shows operational readiness and performance under expected conditions. |
| 7 | TRL 7 - System prototype demonstration in operational environment: A prototype system is demonstrated in an operational environment. The technology is nearing commercialization, with key performance metrics met. |
| 8 | TRL 8 - System complete and qualified: The technology is proven to work in its final form and under expected conditions. It has been demonstrated in real operational settings and meets all requirements for full deployment. |
| 9 | TRL 9 - Actual system proven in operational environment: The technology is fully mature and operational. It has been successfully deployed in a real-world setting, demonstrating full functionality, reliability, and performance. |

Table S4. Scoring guidelines for operational aspects.

| Operational Aspects |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operational Complexity and Ease of Operation |  | Maintenance Requirements |  | Integration with existing AD system |  | Health and safety requirements |  | Handling and Storage |  |
| The practical aspects of how easy operation will be that can reduce the need for intensive training and steps to do the operation. |  | Total required interventions, such as cleaning, inspection, and replacement of parts, which can reduce labour costs and minimize downtime |  | The practical aspects of how many modifications to the current process, the need for additional equipment, and lower capital investment costs |  | The practical aspects of how many potential risks of accidents or environmental hazards when handling chemicals and operating equipment and generation of toxic by-products |  | The practical aspects of storing and managing the substrate. It deals with space requirements, handling equipment, preservation methods, and potential losses during storage. |  |
| Score | Scoring guidelines | Score | Scoring guidelines | Score | Scoring guidelines | Score | Scoring guidelines | Score | Scoring guidelines |
| 1 | Extremely Difficult: Requires working at very high temperatures and pressures with very complex procedures. It needs intensive and extensive training, and the steps are intricate and numerous. | 1 | Extremely High Maintenance: requires constant attention, with continuous and intensive cleaning and almost constant replacement of parts. It demands a lot of manpower and resources. | 1 | Extremely Difficult: Integration requires a complete overhaul, intensive labour, and very high costs for new equipment. | 1 | Very Poor: High risk with many serious safety issues, posing significant health hazards. Requires extensive new health and safety measures throughout the entire process. | 1 | Very Poor: Requires extensive storage facilities, specialised equipment, and high labour costs. Significant losses and quality degradation are common. |
| 2 | Very Difficult: Involves high temperatures and pressures with challenging procedures. Significant training is required, and the operation steps are complex and demanding. | 2 | Very High Maintenance: needs frequent attention, including regular and thorough cleaning and frequent replacement of parts. It significantly affects how smoothly operations run. | 2 | Very Difficult: Integration is tough, needing significant changes and a lot of investment. | 2 | Poor: Notable health risks with several safety issues. Requires additional health and safety measures across most parts of the process. | 2 | Poor: Requires considerable storage facilities and handling efforts, with moderate losses and quality issues. |
| 3 | Difficult: Operation is challenging due to elevated temperatures and pressures. Considerable training is needed, and the steps are complicated but manageable. | 3 | High Maintenance: requires regular maintenance, with consistent cleaning and part replacements. It's demanding, but not as much as the higher levels. | 3 | Difficult: Integration is challenging but doable, requiring substantial changes and considerable costs. | 3 | Fair: Moderate health risks with some safety issues. Health and safety measures are needed in significant parts of the process. | 3 | Fair: Requires adequate storage facilities and handling efforts, with manageable losses and occasional quality issues. |
| 4 | Moderately Difficult: The process involves moderate temperatures and pressures with some complexity. Training is necessary, and the operation includes several steps that need careful attention. | 4 | Moderate Maintenance: needs occasional attention, with periodic cleaning and infrequent part replacements. It is a balanced approach to maintenance. | 4 | Moderate: Integration needs some effort, with moderate adjustments and costs. | 4 | Good: Minimal health risks with few safety issues. Health and safety measures are needed only in some parts of the process. | 4 | Good: Requires reasonable storage facilities and minimal handling efforts, with low losses and good-quality maintenance. |
| 5 | Somewhat Easy: The operation is done at manageable temperatures and pressures, is less complex, and requires some training. The steps are more straightforward. | 5 | Low Maintenance: only needs infrequent attention, with minimal cleaning and rare part replacements. It doesn't demand much from your resources or personnel. | 5 | Somewhat Easy: Integration is relatively simple, with minimal changes and low costs. | 5 | Very Good: Very low health risks with minimal safety issues. Requires only minimal health and safety measures. | 5 | Very Good: Requires minimal storage facilities and handling efforts, with negligible losses and excellent quality maintenance. |
| 6 | Easy: The operation can be done at standard temperatures and pressures, is simple to manage, requires minimal training, and involves fewer, straightforward steps. | 6 | Minimal to No Maintenance: hardly needs any attention, with virtually no cleaning or part replacements required. It's highly efficient and self-sufficient. | 6 | Easy: Integration is seamless, requiring no significant changes or costs. | 6 | Excellent: No health risks or safety issues. No additional health and safety measures are needed; the process can proceed under current conditions. | 6 | Excellent: Requires no additional storage facilities or handling efforts, with no losses and perfect quality and maintenance |

Table S5. Scoring guidelines for economic aspects

| Economic Aspects |  |  |  |
| --- | --- | --- | --- |
| Capital Cost |  | Operational Cost |  |
| Number of financial resources required to implement the technology (E.g.: purchase of land, buildings, construction, and equipment) |  | Expenses which are related to the continuous operation of the technology (E.g.: energy costs such as cost of power for operations and cost of salaries/wages of specialist operators if required) |  |
| Score | Scoring guidelines | Score | Scoring guidelines |
| 1 | Very high: Require purchasing new land, buildings, equipment and construction of plant | 1 | Very high: high energy and labour/consultant cost) |
| 2 | High: Plant construction and purchase of high-cost equipment | 2 | High: high energy cost; low labour cost) |
| 3 | Moderate: Plant construction and purchase of low-cost equipment | 3 | Moderate: low cost for energy; high labour/ consultant cost |
| 4 | Low: Purchase of high-cost equipment | 4 | Low: Low cost for energy; low cost for labour |
| 5 | Minimum: Purchase of equipment; low cost | 5 | Minimum: Low cost for energy; no other operational cost |
| 6 | No requirement of any capital investment | 6 | No operational costs associated |

Table S6. Scoring guidelines for environmental aspects

| Environmental Aspects |  |  |  |
| --- | --- | --- | --- |
| Waste and Wastewater generation |  | Gas emission |  |
| the amount and type of waste generated (including the level of hazard of the chemical handled), also its potential environmental and economic implications. The more hazardous the chemical is, more complex treatment needed before discharging to the environment |  | The amount and type of gaseous emissions produced during the operation of the technology, including greenhouse gases (GHGs) like CO2, CH4, and N2O, as well as other pollutants like NOx, SOx, and volatile organic compounds (VOCs). |  |
| Score | Scoring guidelines | Score | Scoring guidelines |
| 1 | Large amount of waste and highly hazardous: This requires very complex and costly treatment before it can be safely discharged into the environment. | 1 | Very high emissions of GHGs and pollutants: The technology produces a large volume of GHGs and other harmful gases. This level of emission significantly impacts the environment, contributing to climate change and air quality degradation. Requires very complex and costly emission control systems. |
| 2 | Considerable amount of waste and hazardous. This waste requires complex and costly treatment before it can be safely discharged. | 2 | High emissions of GHGs and pollutants: The technology generates a considerable amount of GHGs and other harmful gases. These emissions have a substantial environmental impact and necessitate complex and costly emission control systems. |
| 3 | Moderate waste and moderately hazardous: Standard treatment is required before the waste can be safely discharged. | 3 | Moderate emissions of GHGs and pollutants: The technology emits a moderate volume of GHGs and other harmful gases. Standard emission control systems are required to mitigate the environmental impact. |
| 4 | Low waste but high hazard: Despite the low quantity, complex treatment is necessary before discharge. | 4 | Low emissions of GHGs but high pollutants: The technology produces low amounts of GHGs but significant levels of other harmful gases. Requires specialised treatment to manage the emissions effectively. |
| 5 | Low waste and low hazard: Minimal treatment is required before the waste can be safely discharged. | 5 | Low emissions of GHGs and pollutants: The technology generates a low volume of GHGs and other harmful gases. Minimal emission control measures are needed. |
| 6 | Very low waste or non-hazardous: The technology generates little or no waste. If any waste is generated, it is non-hazardous and requires no treatment before discharge. | 6 | Very low emissions or non-hazardous gases: The technology produces very little or no emissions of GHGs or other harmful gases. Any emissions are non-hazardous and require no special treatment. |

Table S7. Example of the original WSA calculation for each technology

| Technology | Main Criteria | Sub Criteria | Weight | Score | Weighted score |
| --- | --- | --- | --- | --- | --- |
| T1 | A | a1 | w1 | s1 | w1s1 |
|  |  | a2 | w2 | s2 | w2s2 |
|  | B | b1 | w3 | s3 | w3s3 |
|  |  | b2 | w4 | s4 | w4s4 |
|  | C | c1 | w5 | s5 | w5s5 |
|  |  | c2 | w6 | s6 | w6s6 |
|  | D | d1 | w7 | s7 | w7s7 |
|  |  | d2 | w8 | s8 | w8s8 |
|  | Total Weighted Score for T1 |  |  |  | 𝛴WiSi |

Table S8. The scoring guidelines for confidence level calculation of the technology selection process

| Journal Quality (40%) |  |  |  |  |  | Relevance of the research (60%) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Journal Quartal (20%) |  | Citations (10%) |  | h-index of the Correspondence Author (10%) |  | Configuration (20%) |  | The Scale of Experiment (20%) |  | Substrate Relevance (20%) |  |
| The quality of the journal |  | Number of other authors citing the article. |  | Relevance of the author's expertise to the subject. |  | How closely the reactor configuration in the article matches the one intended for my project or a full-scale setup |  | Scale of the study (e.g., lab-scale versus full-scale), as it impacts the applicability of the data. |  | How relevant the substrate used in the study is to my project. |  |
| Score | Scoring guidelines | Score | Scoring guidelines | Score | Scoring guidelines | Score | Scoring guidelines | Score | Scoring guidelines | Score | Scoring guidelines |
| 1 | 5 | 1 | 0-40 | 1 | 1-20 | 1 | Batch | 1 | Lab | 5 | DAF sludge from Dairy industries |
| 2 | 4 | 2 | 41-80 | 2 | 21-40 | 2 | Semi continuous | 2 | Pilot | 4 | DAF sludge from other industries |
| 3 | 3 | 3 | 81-120 | 3 | 41-60 | 3 | Continuous | 3 | Bench | 3 | Lipid Rich wastewater/FOG |
| 4 | 2 | 4 | 121-160 | 4 | 61-80 |  |  | 4 | Plant | 2 | Other dairy products |
| 5 | 1 | 5 | more than 160 | 5 | 81-100 |  |  |  |  | 1 | other type of wastewater |

Table S9. Example for the modified WSA methodology with the confidence level

| Technology | Main Criteria | Sub Criteria | Weight | Score | Confidence Level | Weighted score |
| --- | --- | --- | --- | --- | --- | --- |
| T1 | A | a1 | w1 | s1 | CL1 | w1s1CL1 |
|  |  | a2 | w2 | s2 | CL2 | w2s2CL2 |
|  | B | b1 | w3 | s3 | CL3 | w3s3CL3 |
|  |  | b2 | w4 | s4 | CL4 | w4s4CL4 |
|  | C | c1 | w5 | s5 | CL5 | w5s5CL5 |
|  |  | c2 | w6 | s6 | CL6 | w6s6CL6 |
|  | D | d1 | w7 | s7 | CL7 | w7s7CL7 |
|  |  | d2 | w8 | s8 | CL8 | w8s8CL8 |
|  | Total Weighted Score of T1 |  |  |  |  | WisiCLi |

Table S10. Example of the confidence level matrix

| Literature | Confidence Level Criteria | Weight | Score | Weighted score |
| --- | --- | --- | --- | --- |
| Article 1 | α | ω1 | s1 | ω1s1 |
|  | β | ω2 | s2 | ω2s2 |
|  | γ | ω3 | s3 | ω3s3 |
|  | Maximum Score | ωi | st | ω3st |
|  | Confidence Level of Article 1 |  |  | =ωisiωist |

Table S 11. Example of confidence level calculation for additives technologies under AD enhancement sub-criterion

| No | Step | Examples of actions |  |
| --- | --- | --- | --- |
| 1 | Identification additives from the literature review that can enhance the Anaerobic digestion of sludge and lipid-rich wastewater | Nine additives are identified for the AD enhancement of the DAF sludge. Take 2 examples from these 9 additives: bentonite and biochar |
| 2 | Comprehensive reading on literature of each additive to get how much is the increase of AD caused by the addition of additives | Identification of the sources Sources; Bentonite; Biochar. Article 1; Enhancing performance and stability of anaerobic digestion of chicken manure using thermally modified bentonite; Optimized production conditions and activation of biochar for effective promotion of long-chain fatty acid degradation in anaerobic digestion. Article 2; Low-cost composited accelerants for anaerobic digestion of dairy manure: Focusing on methane yield, digestate utilization and energy evaluation; Anaerobic digestion of fat, oil, and grease (FOG) under combined additives: Enhanced digestibility, biogas production, and microbiome. |
| 3 | Identify of AD enhancement data of bentonite and biochar from 2 articles, | AD enhancement Sources; Bentonite; Biochar. Article 1; 41%; 130%. Article 2; 66.67%; 579%. |
| 4 | Assign the score for each literature based on the CL scoring guidelines | Score of AD enhancement according to the guidelines Sources; Bentonite; Biochar. Article 1; 3; 6. Article 2; 4; 15. |
| 5 | Start the process of CL calculation by identification of the information related to the articles for bentonite and biochar | Information of the sources according to the guidelines Sources; Bentonite; Biochar. Article 1; Journal quartal; 1; 1. h-index; 30; 34. Citations; 58; 17. Scale; lab; lab. Configuration; batch; batch. Substrate; Chicken manure; Synthetic LCFAs. Article 2; Journal quartal; 1; 1. h-index; 53; 42. Citations; 68; 2. Scale; lab; lab. Configuration; batch; batch. Substrate; Dairy manure; FOG. |
| 6 | Determine the score for confidence level of the literatures used based on the scoring guidelines | Scoring the sources according to the guidelines CL Sources; Bentonite; Biochar. Article 1; Journal quartal; 5; 5. h-index; 2; 3. Citations; 2; 1. Scale; 1; 3. Configuration; 1; 1. Substrate; 1; 1. Article 2; Journal quartal; 5; 5. h-index; 3; 3. Citations; 2; 1. Scale; 1; 1. Configuration; 1; 1. Substrate; 1; 3. |
| 7 | Multiply the score for confidence level with weight used for the confidence level's sub-criterion (ωisi) | Multiplication of the score with the weights of CL’s sub-criteria Sources; Weight Raw Score and; Result. Weight; Bentonite; Biochar; Bentonite; Biochar. Article 1; Journal quartal; 20%; 5; 5; 1.0; 1.0. h-index; 10%; 2; 3; 0.2; 0.3. Citations; 10%; 2; 1; 0.2; 0.1. Scale; 20%; 1; 3; 0.2; 0.6. Configuration; 20%; 1; 1; 0.2; 0.2. Substrate; 20%; 1; 1; 0.2; 0.2. Article 2; Journal quartal; 20%; 5; 5; 1.0; 1.0. h-index; 10%; 3; 3; 0.3; 0.3. Citations; 10%; 2; 1; 0.2; 0.1. Scale; 20%; 1; 1; 0.2; 0.2. Configuration; 20%; 1; 1; 0.2; 0,2. Substrate; 20%; 1; 3; 0.2; 0.6. |
| 8 | Sum the result of weighted score (ωisi) for all sub-criteria | Sum result Sources; Bentonite; Biochar. Article 1; 2.0; 2.3. Article 2; 2.1; 2.4. |
| 9 | Calculate the maximum score for CL calculation (the most ideal score) | Maximum weighted score for CL Sub-Criterion of CL; Weight; Score; Weighted score. Journal quartal; 20%; 5; 1.0. h-index; 10%; 5; 0.5. Citations; 10%; 5; 0.5. Scale; 20%; 4; 0.8. Configuration; 20%; 3; 0.6. Substrate; 20%; 5; 1.0. Sum of the ideal score is 4.4 |
| 10 | Normalize the sum of weighted score by dividing it into the ideal score | Final Confidence Level Sources; Bentonite; Biochar. Article 1; 0.45; 0.52. Article 2; 0.48; 0.55. Therefore, the confidence level of that article is around 0.45-0.55 |
| 11 | Multiply the "AD enhancement" score with respective article's confidence level (siCLi) | Multiplication of the AD enhancement score with its respective CL Sources; Score for AD enhancement; Confidence Level for each Article; Result of Multiplication. Bentonite; Biochar; Bentonite; Biochar; Bentonite; Biochar. Article 1; 3; 6; 0.45; 0.52; 1.36; 3.14. Article 2; 4; 15; 0.48; 0.55; 1.91; 8.18. |
| 12 | Calculate the average of each additive's result siCLin | Because each additives have 2 articles as their resources, then n=2 Sources; siCLin. Bentonite; 1.635. Biochar; 5.66. |
| 13 | Multiply the previous result with the weight to get the weighted score for each additive. | Weight for AD enhancement for additives is 40% Sources; wisiCLin. Bentonite; 0.65. Biochar; 2.26. Therefore, according to 2 articles available of bentonite and biochar, the score of the sub-criterion “AD enhancement” is 0.65 and 2.26 respectively. |

Note that in this example, only two articles are used. In the actual calculation, each additive uses more than three articles to get the enhancement data

Table S12. Justification for the scoring the pretreatment technologies under main criterion operational aspects

| Sub-Criteria | Pretreatment Technology | Score | Reason |
| --- | --- | --- | --- |
| Complexity and Ease of Operation | Acid | 4. | Acid pretreatments involve the addition of chemicals to adjust pH. It requires careful monitoring and control of chemical dosages but are generally straightforward to implement. |
|  | Alkaline | 4. | Alkaline pretreatment also involves the addition of chemicals to adjust pH. It also requires careful monitoring and control of chemical dosages but are generally straightforward to implement. |
|  | Biological | 2. | Biological pretreatments using micro-aeration requires more complex systems and careful monitoring of dosing the oxygen to prevent any digester failure, making them more challenging to operate. |
|  | Oxidation | 2. | Oxidation pretreatments often require more complex systems and careful monitoring of multiple parameters, making them more challenging to operate. |
|  | Thermal | 5. | Thermal pretreatment is relatively straightforward, often involving heating the substrate to a specific temperature for a set duration. It's generally easy to control and monitor. |
|  | Enzymatic | 3. | Enzymatic pretreatment can be more complex due to the need for specific enzymes and controlled conditions, but modern systems have simplified the process. |
|  | Thermochemical | 1. | Thermochemical pretreatment combines thermal and chemical processes, making it the most complex to operate and control among these options. |
|  | Ultrasonic | 3. | Ultrasonic pretreatment requires specialized equipment but is relatively easy to operate once set up correctly. |
| Maintenance Requirements | Acid | 3. | Higher maintenance needs due to corrosiveness, primarily involving regular checking and cleaning of dosing systems or air diffusers. |
|  | Alkaline | 4. | primarily involving regular checking and cleaning of dosing systems. |
|  | Biological | 4. | primarily involving regular checking and cleaning of dosing systems or air diffusers. |
|  | Oxidation | 2. | often involve more complex equipment that requires more frequent inspection, cleaning, and potential part replacements. |
|  | Thermal | 6. | Thermal pretreatment generally has low maintenance requirements, mainly involving regular inspection and cleaning of heat exchangers or boilers. |
|  | Enzymatic | 3. | Enzymatic pretreatment may require more frequent interventions to ensure proper enzyme dosing and activity. |
|  | Thermochemical | 1. | Thermochemical pretreatment typically has the highest maintenance requirements due to the combination of thermal and chemical processes |
|  | Ultrasonic | 2. | often involve more complex equipment that requires more frequent inspection, cleaning, and potential part replacements. |
| Health and Safety Requirement | Acid | 4. | Acid pretreatments involve handling corrosive chemicals, requiring proper safety measures and protective equipment. |
|  | Alkaline | 4. | Alkaline pretreatments involve handling caustic chemicals, requiring proper safety measures and protective equipment. |
|  | Biological | 5. | Biological pretreatment is generally the safest as it uses natural processes and microorganisms, posing minimal risks to human health and the environment. |
|  | Oxidation | 2. | Oxidation often involves strong oxidizing agents, which can be hazardous if not handled properly. |
|  | Thermal | 6. | Thermal pretreatment involves heat, which can pose some safety risks, but these are generally well-understood and manageable. |
|  | Enzymatic | 5. | Enzymatic pretreatment is relatively safe, with enzymatic using biological catalysts with minimal chemical exposure risks |
|  | Thermochemical | 1. | Thermochemical pretreatment combines high temperatures with chemical treatments, posing the highest potential risks among these options |
|  | Ultrasonic | 4. | Ultrasonic pretreatments are relatively safe, with sound waves. It has minimal chemical exposure risks |
| Integration with existing AD system | Acid | 2. | Alkaline and Acid pretreatments require additional chemical storage and dosing systems but can generally be integrated without major modifications to the main AD process. |
|  | Alkaline | 3. | Alkaline and Acid pretreatments require additional chemical storage and dosing systems but can generally be integrated without major modifications to the main AD process. |
|  | Biological | 6. | Micro-aeration as a biological pretreatment method is relatively easy to integrate into existing AD systems. It typically requires only the addition of a controlled air or oxygen injection system, which can often be incorporated into existing pipework or tanks with minimal modifications. This method doesn't usually require separate reactors or extensive changes to the main AD process, making it much more integration-friendly than other biological pretreatment methods. |
|  | Oxidation | 2. | Oxidation often requires specialized equipment and safety measures, which can be more challenging to integrate into existing systems. |
|  | Thermal | 4. | Thermal pretreatment is often the easiest to integrate as it primarily requires heating equipment, which can often be incorporated into existing systems with minimal modifications. |
|  | Enzymatic | 5. | Enzymatic pretreatment can be relatively easy to integrate as standalone units before the AD process, with minimal impact on the existing system. |
|  | Thermochemical | 1. | Thermochemical pretreatment typically requires significant additional equipment and process modifications, making it the most challenging to integrate. |
|  | Ultrasonic | 5. | Ultrasonic pretreatment requires specialized high-power equipment, reaction chambers that withstand cavitation, systems to control substrate flow, substantial energy input, and process control for optimization. These needs introduce challenges such as finding space for the equipment, making significant piping modifications, handling increased energy costs, managing more operational parameters, and ensuring maintenance with specialized expertise. |

Table S13. Justification for the scoring the pretreatment technologies under main criterion economic aspects

| Sub-Criteria | Pretreatment Technology | Score | Reason |
| --- | --- | --- | --- |
| Capital Cost | Acid | 5. | requires minimal additional equipment, mainly storage tanks and dosing systems. chemical pretreatments generally have lower capital costs compared to other pretreatment methods. But acid pretreatment needs to instal corrosive resistant materials. |
|  | Alkaline | 6. | require minimal additional equipment, mainly storage tanks and dosing systems. chemical and biological pretreatments generally have lower capital costs compared to other methods. But acid need to instal corrosive resistant materials, so alkaline is higher |
|  | Biological | 6. | Low equipment requirements: Micro-aeration typically requires minimal additional equipment, mainly consisting of air pumps or blowers and diffusers. This simplicity contributes to its low capital cost. |
|  | Oxidation | 1. | This technology often involves complex systems and multiple process steps. Advanced oxidation processes, particularly ozonation, have high capital costs due to the specialized equipment required. |
|  | Thermal | 3. | This technology involves especially high-temperature processes that require specialized reactors and safety systems. |
|  | Enzymatic | 6. | require minimal additional equipment, mainly storage tanks and dosing systems, chemical and biological pretreatments generally have lower capital costs compared to other methods. |
|  | Thermochemical | 1. | This technology combines the requirement of thermal and chemical pretreatment into a system, consequently, consideration of supplying the heat and chemical resistance materials is required |
|  | Ultrasonic | 4. | These technologies require more specialized equipment but are still relatively compact and can be integrated into existing systems without major modifications. |
| Operational Cost | Acid | 4. | These chemical pretreatments have moderate operational costs due to the ongoing need for chemical reagents |
|  | Alkaline | 4. | These chemical pretreatments have moderate operational costs due to the ongoing need for chemical reagents. |
|  | Biological | 6. | Biological pretreatment typically has very low operational costs as it requires minimal energy input and no chemical additions. |
|  | Oxidation | 2. | This technology often involves complex systems and multiple process steps. advanced oxidation processes, particularly ozonation, has high operational costs due to the dosing of reagent required. |
|  | Thermal | 5. | Thermal pretreatment has moderate operational costs, mainly due to energy requirements for heating. while thermal pretreatment requires energy input, its operational costs can be offset by increased biogas production." |
|  | Enzymatic | 3. | Enzymatic pretreatment often has higher operational costs due to the expense of enzymes |
|  | Thermochemical | 1. | This technology combines the requirement of thermal and chemical pretreatment into a system, consequently, dosing of chemicals and energy supplies are continuously required |
|  | Ultrasonic | 6. | Ultrasonic pretreatment, while requiring some energy, is generally considered low in operational costs. |

Table S14. Justification for the scoring the pretreatment technologies under main criterion environmental aspects

| Sub-Criteria | Pretreatment Technology | Score | Reason |
| --- | --- | --- | --- |
| Waste and wastewater generation | Acid | 2. | These chemical pretreatments often produce significant amounts of wastewater that require neutralization and treatment. |
|  | Alkaline | 2. | These chemical pretreatments often produce significant amounts of wastewater that require neutralization and treatment. |
|  | Biological | 5. | Biological pretreatment generally produces minimal additional waste, though some excess biomass may be generated. |
|  | Oxidation | 3. | Oxidation processes can produce moderate amounts of wastewater, depending on the specific method used. |
|  | Thermal | 6. | typically produce minimal additional waste or wastewater, as they primarily involve physical changes to the substrate. |
|  | Enzymatic | 4. | Enzymatic pretreatment produces relatively little waste, though there may be some residual enzyme-containing wastewater. |
|  | Thermochemical | 1. | Thermochemical pretreatment often produces the most waste and wastewater due to the combination of chemical and thermal processes. |
|  | Ultrasonic | 6. | typically produce minimal additional waste or wastewater, as they primarily involve physical changes to the substrate. |
| Gas emission | Acid | 4. | Acid pretreatments may produce some odorous compounds, but overall gas emissions are moderate compared to thermal methods |
|  | Alkaline | 4. | Alkaline pretreatments may produce some odorous compounds, but overall gas emissions are moderate compared to thermal methods |
|  | Biological | 5. | While biological pretreatment may generate some biogas, the emissions are typically low and can often be captured for beneficial use. |
|  | Oxidation | 3. | Advanced oxidation processes may generate gaseous by-products, including volatile organic compounds, depending on the substrate and oxidation method. |
|  | Thermal | 2. | GHG emission for this pretreatment is higher due to the required energy for heating, also it can result in the release of volatile organic compounds and odorous substances, particularly at higher temperatures |
|  | Enzymatic | 6. | Enzymatic methods are generally conducted at mild conditions and do not generate significant gaseous emissions. |
|  | Thermochemical | 1. | Emission can come from the energy requirement for the heating, Thermochemical pretreatment methods can generate substantial gaseous emissions, including CO2, volatile organic compounds, and potentially hazardous gases depending on the feedstock and conditions |
|  | Ultrasonic | 6. | Ultrasonic pretreatment does not directly produce gaseous emissions, making it environmentally friendly in terms of air quality. |

Table S15. Justification for the scoring the co-substrate for the anaerobic co-digestion technologies

| Sub-Criteria | Co-substrate | Score | Reason |
| --- | --- | --- | --- |
| Complexity and Ease of Operation | Agricultural Biomass | 2. | Often requires significant pretreatment (size reduction, potentially hydrolysis), seasonal availability may complicate operations, may require specialized handling equipment |
|  | Manure | 5. | Simple handling and feeding processes |
|  | Slaughterhouse Waste | 3. | Requires careful handling due to pathogen concerns, may need hygienization, composition can vary significantly |
|  | Sewage Sludge | 5. | Often integrated into existing wastewater treatment plants, continuous availability, relatively simple feeding process, and may require some monitoring for composition variations |
|  | Waste Activated Sludge | 5. | Like sewage sludge in terms of integration and availability, may require monitoring for microbial activity, potential for direct feeding into digesters |
|  | Food waste | 4. | Requires sorting and potential grinding, composition can vary, requiring monitoring, and may need management of seasonality and supply chain |
| Maintenance Requirements | Agricultural Biomass | 3. | Agricultural Biomass or lignocellulosic biomass generally requires minimal preprocessing, but due to the fibre complex structure, it has potential for causing system blockages |
|  | Manure | 5. | Because the remaining fibre complex structure that are still in the substrate, it has potential for causing system blockages |
|  | Slaughterhouse Waste | 2. | Slaughterhouse Waste can be variable in composition and may contain high levels of proteins and fats, potentially leading to foaming issues and more frequent cleaning requirements. |
|  | Sewage Sludge | 3. | This substrate is usually well-characterized but may require more frequent cleaning of heat exchangers and pipes due to potential scaling. |
|  | Waste Activated Sludge | 5. | This substrate is usually well-characterized but may require more frequent cleaning of heat exchangers and pipes due to potential scaling. |
|  | Food waste | 2. | Food waste is highly variable in composition, often requiring pretreatment (sorting, grinding), and can lead to more frequent system cleaning due to potential for clogging and foaming. |
| Health and Safety Requirement | Agricultural Biomass | 2. | This substrate is relatively easy to handle and store due to its dry nature and low moisture content. It can be stored for extended periods without significant degradation. |
|  | Manure | 4. | Manure is relatively easy to handle and can be stored for moderate periods. However, it requires proper management to control odours and prevent nutrient runoff. |
|  | Slaughterhouse Waste | 3. | This substrate is challenging to handle and store due to its high moisture content, rapid degradation, and potential for pathogen growth. It requires careful management and often needs to be processed quickly. |
|  | Sewage Sludge | 5. | Sewage sludge can be challenging to handle and store due to its high-water content and potential for odour issues. However, dewatering techniques can improve its handling characteristics. |
|  | Waste Activated Sludge | 6. | waste activated sludge presents challenges in handling and storage due to its high moisture content. Dewatering is often necessary for improved management. |
|  | Food waste | 4. | Food waste is one of the most challenging substrates to handle and store due to its high moisture content, rapid degradation, and potential for odour and pest issues. |
| Integration with existing AD system | Agricultural Biomass | 5. | Often requires significant additional pretreatment equipment, may need specialized handling and feeding systems, and integration can be more complex due to physical characteristics |
|  | Manure | 4. | Relatively easy to integrate, especially in farm-based AD systems, may require minimal additional equipment for handling and feeding |
|  | Slaughterhouse Waste | 2. | May require additional handling and hygienization equipment, potential need for storage and pretreatment facilities, and it can be integrated but with more considerations than simpler substrates |
|  | Sewage Sludge | 3. | Often already part of existing wastewater treatment plants with AD systems, minimal additional infrastructure required for integration, and well-established co-digestion practices with FOG |
|  | Waste Activated Sludge | 5. | like sewage sludge in terms of integration ease, it is often produced on-site in wastewater treatment plants, existing handling and feeding systems can be utilized |
|  | Food waste | 2. | Requires some additional pretreatment equipment (e.g., sorting, grinding), and it may need storage facilities |
| Handling and storage | Agricultural Biomass | 4. | It can be stored for extended periods if properly managed, it may require large storage areas, and its handling can be mechanized but may require specialized equipment |
|  | Manure | 3. | Simple handling requirements, relatively stable during storage, and it is well-established handling practices |
|  | Slaughterhouse Waste | 2. | Requires careful handling due to pathogen concerns, it often needs refrigeration or other preservation methods, and it may have odour issues and attract vermin if not properly managed |
|  | Sewage Sludge | 4. | it is typically managed within existing wastewater treatment infrastructure, continuously produced to minimize long-term storage needs, and handled according to established protocols |
|  | Waste Activated Sludge | 4. | Like sewage sludge, it is typically managed within wastewater treatment plants, and its continuous production minimizes long-term storage needs |
|  | Food waste | 3. | It requires careful storage to prevent odours and vermin, may need refrigeration or other preservation methods, and handling can be messy, necessitating specialized equipment |
| Capital Cost | Agricultural Biomass | 2 | To have a fully optimized digestion, it may require more extensive pretreatment (e.g., size reduction, potentially hydrolysis), increasing capital cost |
|  | Manure | 5 | Manure typically requires minimal pretreatment and is often readily available at farm-scale AD plants, resulting in lower capital costs. |
|  | Slaughterhouse Waste | 3 | Often requires more robust handling and pretreatment systems due to its nature, increasing capital costs. |
|  | Sewage Sludge | 5 | In wastewater treatment plants, it might necessary infrastructure be already existing, requiring minimal additional pretreatment equipment for co-digestion with FOG |
|  | Waste Activated Sludge | 5 | Existing wastewater treatment infrastructure can often be utilized, allowing for direct feeding into digesters without extensive additional equipment |
|  | Food waste | 4 | While some pretreatment may be necessary (e.g., sorting, grinding), the equipment is generally not that expensive |
| Operational Cost | Agricultural Biomass | 3 | It requires higher energy for size reduction and potential pretreatment, may need specialized equipment for dosing, increasing labour and energy costs, and potential seasonal availability affecting labour planning |
|  | Manure | 6 | Often available on-site, reducing transportation energy, and has a simple dosing process with minimal labour. |
|  | Slaughterhouse Waste | 2 | It requires moderate to high energy for pretreatment and sanitation, careful handling and dosing that increase labour costs, and may need heated storage, raising energy costs. |
|  | Sewage Sludge | 4 | Feeding WAS directly into anaerobic digesters without complex pretreatment is possible. Existing infrastructure will help reducing operational complexity and costs. Additionally, continuous production of these sludges simplifies operations compared to seasonally variable substrates. |
|  | Waste Activated Sludge | 4 | Direct feeding is possible as many wastewaters treatment plants feed sewage sludge directly into anaerobic digesters without complex pretreatment. Existing infrastructure will help reducing operational complexity and costs. Additionally, continuous production of these sludges simplifies operations compared to seasonally variable substrates. |
|  | Food waste | 5 | It requires moderate energy for pretreatment such as sorting and grinding, may need more labour for sorting and contaminant removal, but has a relatively simple dosing process. |
| Waste and wastewater generation | Agricultural Biomass | 3 | While these materials can be effectively co-digested with FOG, they may require additional pretreatment and can generate more waste undigested residues due to their complex structure. |
|  | Manure | 6 | Manure, especially when properly managed, Manure-based digestate is excellent for fertilizer use, rich in nutrients, and generally safe for agricultural application. It effectively closes the nutrient cycle. |
|  | Slaughterhouse Waste | 3 | Digestate from slaughterhouse waste can be nutrient-rich but may face regulatory challenges and require additional treatment before use as fertilizer. |
|  | Sewage Sludge | 4 | While sewage sludge digestate can be used as fertilizer, it often faces stricter regulations due to potential contaminants. It may require additional treatment or have limited application. |
|  | Waste Activated Sludge | 4 | When WAS is co-digested with FOG, the properties of the resulting digestate can be significantly different from digestate produced from WAS alone. Co-digestion often improves the quality and usability of the digestate. |
|  | Food waste | 4 | While food waste digestate is nutrient-rich, it may require more careful management due to potential variability in composition and salt content. |
| Gas emission | Agricultural Biomass | 5 | This substrate typically has low moisture content and a high C/N ratio, which results in lower odour emissions and reduced risk of ammonia release. When co-digested with lipids, it can help balance the C/N ratio and reduce the risk of inhibitory effects from long-chain fatty acids (LCFAs). |
|  | Manure | 3 | Manure can produce significant odour and ammonia emissions due to its high nitrogen content. However, when co-digested with lipids, it can provide a good nutrient balance and help mitigate some of the inhibitory effects of LCFAs. |
|  | Slaughterhouse Waste | 2 | This substrate can produce strong odours and potentially high levels of H2S due to its high protein and sulphur content. It may also lead to increased ammonia emissions. Co-digestion with lipids can exacerbate these issues if not properly managed. |
|  | Sewage Sludge | 3 | Sewage sludge can produce odorous compounds and has the potential for higher H2S emissions. However, when co-digested with lipids, it can provide a good nutrient balance and help mitigate some emission issues. |
|  | Waste Activated Sludge | 4 | Waste activated sludge generally has lower odour potential compared to primary sludge and can help balance nutrient ratios when co-digested with lipids. It may also contribute to reduced H2S emissions due to the presence of sulphur-oxidizing bacteria. |
|  | Food waste | 3 | Food waste can produce significant odour emissions and has the potential for high VFA production, which can lead to process instability and increased CO2 emissions. However, when properly balanced with lipids, it can provide good biogas yields and help mitigate some emission issues. |

Table S16. Justification for the scoring the additives technologies

| Sub-Criteria | Co-substrate | Score | Reason |
| --- | --- | --- | --- |
| Complexity and Ease of Operation | Activated carbon | 5 | Easy to handle and dose, typically requiring only basic operational adjustments. Relatively straightforward to use but may require more precise dosing and monitoring. |
|  | Biochar | 4 | Relatively straightforward to use but may require more precise dosing and monitoring. |
|  | Carbon cloth | 3 | May require more specialized handling and operational procedures. |
|  | Enzymes | 4 | Relatively straightforward to use but may require more precise dosing and monitoring. |
|  | Graphene | 2 | This nanomaterial often requires complex handling procedures and sophisticated operational controls. |
|  | Magnetite | 3 | May require more specialized handling and operational procedures. |
|  | Supplementation | 4 | Generally simple to add and requires minimal operational changes. |
|  | Bentonite | 5 | Easy to handle and dose, typically requiring only basic operational adjustments. |
|  | Carbon Nano Tube (CNT) | 1 | These nanomaterials often require complex handling procedures and sophisticated operational controls. |
| Maintenance Requirements | Activated carbon | 4 | Relatively easy to add and typically don't cause significant maintenance issues. May require occasional replenishment. |
|  | Biochar | 4 | Generally low maintenance but may require more frequent inspection to ensure even distribution and prevent accumulation. |
|  | Carbon cloth | 3 | May require more frequent inspection and potential replacement. Could potentially cause some accumulation or clogging issues. |
|  | Enzymes | 4 | Relatively easy to add and typically don't cause significant maintenance issues. May require occasional replenishment. |
|  | Graphene | 2 | These advanced materials may require more specialized handling, potential for agglomeration, and more frequent inspection/replacement. |
|  | Magnetite | 3 | May require more frequent inspection and potential replacement. Could potentially cause some accumulation or clogging issues. |
|  | Supplementation | 6 | Supplementation (Co, Fe, and Ni): Generally, requires minimal maintenance as these are typically added in small quantities and don't accumulate significantly. |
|  | Bentonite | 4 | Generally low maintenance but may require more frequent inspection to ensure even distribution and prevent accumulation. |
|  | Carbon Nano Tube (CNT) | 1 | These advanced materials may require more specialized handling, potential for agglomeration, and more frequent inspection/replacement. |
| Health and Safety Requirement | Activated carbon | 5 | This material is generally safe but may pose some inhalation risks if fine particles are generated during handling. |
|  | Biochar | 5 | This material is generally safe but may pose some inhalation risks if fine particles are generated during handling. |
|  | Carbon cloth | 5 | This material is generally safe but may pose some inhalation risks if fine particles are generated during handling. |
|  | Enzymes | 3 | While many enzymes are safe, some may cause allergic reactions or respiratory sensitization in certain individuals. |
|  | Graphene | 2 | Emerging research suggests potential health risks, particularly from inhalation of graphene nanoparticles. |
|  | Magnetite | 4 | Magnetite is generally safe but may pose risks if inhaled in large quantities |
|  | Supplementation | 4 | Supplementation safety depends on the specific nutrients used but is generally safe when handled properly. |
|  | Bentonite | 6 | Generally recognized as safe (GRAS) by the FDA. It's a natural clay with minimal health risks when handled properly. |
|  | Carbon Nano Tube (CNT) | 1 | Studies have shown potential serious health risks, including possible carcinogenic effects, especially when inhaled. |
| Integration with existing AD system | Activated carbon | 5 | Can be added as a powder or granules, typically requiring only minor modifications to feeding systems. |
|  | Biochar | 4 | Generally easy to integrate but may require some modifications to feeding systems and potential adjustments to mixing. |
|  | Carbon cloth | 3 | May require more significant modifications to the AD system, including specialized feeding mechanisms or reactor designs. |
|  | Enzymes | 5 | Can be added directly to the feedstock or digester, requiring minimal system modifications. |
|  | Graphene | 2 | These nanomaterials often require specialized handling and integration techniques, potentially necessitating significant system modifications. |
|  | Magnetite | 3 | May require more significant modifications to the AD system, including specialized feeding mechanisms or reactor designs. |
|  | Supplementation | 6 | Easily integrated into existing AD systems, often through simple dosing systems. |
|  | Bentonite | 4 | Generally easy to integrate but may require some modifications to feeding systems and potential adjustments to mixing. |
|  | Carbon Nano Tube (CNT) | 1 | These nanomaterials often require specialized handling and integration techniques, potentially necessitating significant system modifications. |
| Handling and storage | Activated carbon | 5 | Typically, easy to handle and store, but may require protection from moisture. |
|  | Biochar | 4 | Relatively straightforward to handle and store but may require protection from moisture and contamination. |
|  | Carbon cloth | 4 | Relatively straightforward to handle and store but may require protection from moisture and contamination. |
|  | Enzymes | 3 | May require more specialized storage conditions. Enzymes often need refrigeration and protection from heat and moisture. |
|  | Graphene | 2 | These nanomaterials often require complex handling procedures and sophisticated storage conditions to prevent agglomeration and ensure safety. |
|  | Magnetite | 3 | May require more specialized storage conditions. Enzymes often need refrigeration and protection from heat and moisture. |
|  | Supplementation | 5 | Typically, easy to handle and store, but may require protection from moisture. |
|  | Bentonite | 6 | Generally easy to handle and store, requiring only dry conditions. |
|  | Carbon Nano Tube (CNT) | 1 | These nanomaterials often require complex handling procedures and sophisticated storage conditions to prevent agglomeration and ensure safety. |
| Capital Cost | Activated carbon | 4 | Moderately priced, with costs varying based on quality and source. |
|  | Biochar | 4 | Moderately priced, with costs varying based on quality and source. |
|  | Carbon cloth | 3 | Higher costs due to processing requirements or specialized manufacturing. |
|  | Enzymes | 2 | Can be expensive due to production and purification costs. |
|  | Graphene | 1 | Generally, the most expensive options due to complex manufacturing processes. |
|  | Magnetite | 3 | Higher costs due to processing requirements or specialized manufacturing. |
|  | Supplementation | 5 | Typically, low-cost but can vary depending on the specific nutrients used. |
|  | Bentonite | 6 | Generally, the least expensive option due to its natural abundance. |
|  | Carbon Nano Tube (CNT) | 1 | Generally, the most expensive options due to complex manufacturing processes. |
| Operational Cost | Activated carbon | 4 | Moderate dosage requirements and may need periodic replenishment. |
|  | Biochar | 4 | Moderate dosage requirements and may need periodic replenishment. |
|  | Carbon cloth | 3 | Lower operational costs than nanomaterials but may require periodic replacement. |
|  | Enzymes | 2 | Can be expensive due to high production costs and may require frequent addition. |
|  | Graphene | 1 | Highest operational costs due to high material costs and potential need for specialized handling and monitoring. |
|  | Magnetite | 5 | Typically, low dosage requirements, but may need more frequent addition than bentonite. |
|  | Supplementation | 5 | Typically, low dosage requirements, but may need more frequent addition than bentonite. |
|  | Bentonite | 6 | Generally low dosage requirements and infrequent addition needed. |
|  | Carbon Nano Tube (CNT) | 1 | Highest operational costs due to high material costs and potential need for specialized handling and monitoring. |
| Waste and wastewater generation | Activated carbon | 4 | Moderate waste generation, may require periodic removal and replacement. |
|  | Biochar | 4 | Moderate waste generation, may require periodic removal and replacement. |
|  | Carbon cloth | 3 | May generate more waste due to replacement needs. |
|  | Enzymes | 3 | May generate more waste due residual enzyme products. |
|  | Graphene | 2 | Potential for higher waste generation due to specialized handling and disposal requirements. |
|  | Magnetite | 5 | Generally low waste generation, often integrated into the digestate. |
|  | Supplementation | 5 | Generally low waste generation, often integrated into the digestate. |
|  | Bentonite | 6 | Minimal waste generation, typically incorporated into digestate. |
|  | Carbon Nano Tube (CNT) | 1 | Potential for higher waste generation due to specialized handling and disposal requirements. |
| Gas emission | Activated carbon | 6 | Excellent for adsorption of various gases, including odorous compounds. |
|  | Biochar | 5 | Good potential for gas adsorption and emission reduction. |
|  | Carbon cloth | 5 | Good potential for gas adsorption and emission reduction. |
|  | Enzymes | 3 | May indirectly reduce emissions by improving process efficiency, but effects can vary. |
|  | Graphene | 2 | Limited data on their impact on gas emissions in AD; potential concerns about their own environmental impacts. |
|  | Magnetite | 4 | Can help in reducing some emissions, particularly through improved process stability. |
|  | Supplementation | 3 | May indirectly reduce emissions by improving process efficiency, but effects can vary. |
|  | Bentonite | 4 | Can help in reducing some emissions, particularly through improved process stability. |
|  | Carbon Nano Tube (CNT) | 2 | Limited data on their impact on gas emissions in AD; potential concerns about their own environmental impacts. |

Table S17. Justification for the scoring the process modification technologies

| Sub-Criteria | Process Modification | Score | Reason |
| --- | --- | --- | --- |
| Complexity and Ease of Operation | Two-stage AD | 3 | Two-stage AD offers more control over the digestion process, but it's inherently more complex due to the need to manage two separate reactors with different optimal conditions. This increased complexity can make it more challenging to operate, especially for less experienced operators. |
|  | Thermophilic | 4 | Thermophilic AD is relatively straightforward to operate once the system is established. However, it requires careful temperature control and can be more sensitive to fluctuations in operating conditions compared to mesophilic systems. |
| Maintenance Requirements | Two-stage AD | 2 | It involves two separate reactors, essentially doubling the equipment that needs maintenance. This setup may require more frequent cleaning and inspection due to the separation of acidogenesis and methanogenesis stages. Additionally, potentially more complex control systems increase the maintenance of monitoring and control equipment. However, it may offer advantages in process stability, potentially reducing some types of maintenance interventions. |
|  | Thermophilic | 3 | It operates at higher temperatures (50-60°C), leading to increased wear on equipment and may require more frequent cleaning due to faster reaction rates and potential for rapid fouling. Careful temperature control is needed, potentially increasing maintenance on heating systems. However, its generally simpler design compared to two-stage systems can reduce some maintenance needs. |
| Health and Safety Requirement | Two-stage AD | 5 | Two-stage AD typically operates at lower temperatures in at least one stage, and its separated processes can allow for better control of potentially hazardous conditions. This can lead to a slightly better overall health and safety profile. |
|  | Thermophilic | 4 | Thermophilic AD operates at higher temperatures, which can present some additional safety considerations. However, it also provides better pathogen reduction, which can be a health benefit. Thermophilic AD, while effective, operates at higher temperatures (typically 50-60°C), which increases the risk of burns and requires more careful handling. However, its higher operating temperature also provides better pathogen reduction, which can be a significant health benefit, especially when dealing with waste that may contain pathogens. |
| Integration with existing AD system | Two-stage AD | 3 | Two-stage AD is more challenging to integrate with existing systems, as it often requires significant modifications to the reactor configuration and the addition of a new reactor stage |
|  | Thermophilic | 5 | Thermophilic AD generally integrates more easily with existing AD systems, especially if they are already single-stage systems. The main modifications typically involve upgrading heating systems and potentially reinforcing insulation. |
| Capital Cost | Two-stage AD | 2 | Two-stage AD typically has higher capital costs due to the need for separate reactors for the acidogenic and methanogenic phases, additional control systems, and more complex piping and pumping arrangements. |
|  | Thermophilic | 4 | Thermophilic AD generally has moderate capital costs. While it requires additional heating equipment to maintain higher temperatures, it's a single-stage process that doesn't require the extra infrastructure of two-stage systems. |
| Operational Cost | Two-stage AD | 5 | Two-stage AD typically has lower operational costs despite its higher capital costs. The separation of acidogenesis and methanogenesis phases allows for optimized conditions in each reactor, potentially leading to higher overall efficiency, better stability, and reduced chemical/additive requirements. |
|  | Thermophilic | 4 | Thermophilic AD generally has moderate operational costs. While it requires more energy for heating to maintain higher temperatures (50-60°C), it often achieves faster reaction rates and higher biogas yields, which can partially offset the increased energy input. |
| Waste and wastewater generation | Two-stage AD | 5 | Two-stage AD typically performs better in terms of waste and wastewater generation. The separation of hydrolysis/acidogenesis and methanogenesis phases allows for more complete degradation of organic matter, resulting in less residual waste and often higher quality effluent. |
|  | Thermophilic | 5 | Thermophilic AD generally produces less waste and wastewater compared to mesophilic systems due to higher degradation rates and improved pathogen reduction. However, it may still generate some residual waste and wastewater that requires further treatment or disposal. |
| Gas emission | Two-stage AD | 5 | Two-stage AD typically performs better in terms of overall gas emissions. The separation of hydrolysis/acidogenesis and methanogenesis phases allows for better control of the process, potentially leading to higher methane yields, lower CO2 emissions, and reduced odour issues. |
|  | Thermophilic | 4 | Thermophilic AD generally has good performance in terms of gas emissions. The higher operating temperature leads to faster reaction rates and can result in more complete degradation of organic matter, potentially reducing odorous emissions. However, it may have slightly higher CO2 emissions due to the energy required for heating. |

Table S 18. Scoring on how well the technological solutions can overcome the challenges

| Technological solutions | Lipid digestibility problems and low hydrolysis rate leading to LCFA accumulation | Inhibition leading to low methanogenesis rates (ammonia and VFAs) | Nutrient and chemical balance concerns causing microbial imbalance | Operational issues due to process instability | Variability in dairy processing wastewater | Odor issues | Total | Calculated score in 6-point scale |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Acid | 1 | 0 | 0 | 0 | 0 | 0 | 1 | 0.71 |
| Alkaline | 2 | 0.9 | 0.8 | 0.5 | 0 | 0.45 | 4.65 | 3.28 |
| Microaeration | 1 | 1.8 | 1.6 | 1 | 0 | 0.9 | 6.3 | 4.45 |
| Oxidation | 1 | 0.9 | 0.8 | 0.5 | 0 | 0.9 | 4.1 | 2.89 |
| Hydrothermal | 2 | 0.9 | 0 | 0.5 | 0 | 0.45 | 3.85 | 2.72 |
| Enzymatic | 2 | 0 | 0 | 0.5 | 0 | 0 | 2.5 | 1.76 |
| Thermochemical | 2 | 0.9 | 0.8 | 1 | 0 | 0.45 | 5.15 | 3.64 |
| Ultrasonic | 1 | 0.9 | 0.8 | 0.5 | 0 | 0.45 | 3.65 | 2.58 |
| Agricultural Biomass | 1 | 0.9 | 0.8 | 0.5 | 0.6 | 0.45 | 4.25 | 3.00 |
| Manure | 2 | 0.9 | 1.6 | 1 | 1.2 | 0 | 6.7 | 4.73 |
| Slaughterhouse Waste | 1 | 0 | 0.8 | 0 | 0 | 0 | 1.8 | 1.27 |
| Sewage Sludge | 1 | 0.9 | 0.8 | 0.5 | 0.6 | 0.45 | 4.25 | 3.00 |
| Waste Activated Sludge | 1 | 1.8 | 1.6 | 1 | 1.2 | 0.45 | 7.05 | 4.98 |
| Food Waste | 1 | 0.9 | 0.8 | 0.5 | 0.6 | 0.45 | 4.25 | 3.00 |
| Activated Carbon | 1 | 1.8 | 0.8 | 0.5 | 0 | 0.9 | 5 | 3.53 |
| Biochar | 1 | 1.8 | 1.6 | 0.5 | 0 | 0.9 | 5.8 | 4.09 |
| Carbon Cloth | 0 | 0.9 | 0 | 0 | 0 | 0 | 0.9 | 0.64 |
| Enzymes | 2 | 0 | 0 | 0.5 | 0 | 0 | 2.5 | 1.76 |
| Graphene | 0 | 0.9 | 0 | 0 | 0 | 0 | 0.9 | 0.64 |
| Magnetite | 1 | 0.9 | 0.8 | 0.5 | 0 | 0.45 | 3.65 | 2.58 |
| Supplementation | 1 | 0.9 | 1.6 | 0 | 0 | 0 | 3.5 | 2.47 |
| Bentonite | 1 | 1.8 | 0.8 | 1 | 0 | 0.9 | 5.5 | 3.88 |
| Carbon Nanotubes | 0 | 0.9 | 0 | 0 | 0 | 0 | 0.9 | 0.64 |
| Thermophilic Anaerobic Digestion (TAD) | 2 | 0.9 | 0.8 | 0 | 0.6 | 0.45 | 4.75 | 3.35 |
| Two-Stage Anaerobic Digestion (TSAD) | 2 | 1.8 | 1.6 | 1 | 1.2 | 0.9 | 8.5 | 6.00 |

Figure S 1. Flow diagram of the confidence level calculation for each literature