Enhancing anaerobic co-digestion of poultry litter and wheat straw via integrated microaeration, biochar, and nanoparticle strategies: Kinetic, microbial, and economic insights

Yuanhang Zhan1, Yiting Xiao1, Sarah Wu2, Zhiwu Wang3, Jun Zhu1*

1Department of Biological and Agricultural Engineering, University of Arkansas, Fayetteville, AR 72701, USA

2Department of Chemical and Biological Engineering, University of Idaho, Moscow, ID 83844, USA

3Department of Biological Systems Engineering, Virginia Tech, Blacksburg, VA 24060, USA

*Correspondence: yz062@uark.edu; Tel.: +1-479-575-2688.

Fig. S1. The scanning electron microscope (SEM) images of the (a1-a3) biochar and (b1-b3) magnetite nanoparticles at different magnitudes

Fig. S2. The variation of (a) methane content (%) and (b) daily methane production (mL CH4) of the Co-AD process under different conditions (C, the control; M, microaeration; B, biochar addition; N, nanoparticles; and their combinations)

Table S1 Summary of the kinetic parameters using the first-order kinetic model (FM), the modified Gompertz model (MGM) for the Co-AD process under different conditions (C, the control; M, microaeration; B, biochar addition; N, nanoparticles; and their combinations).

| Groups |  | C | M | B | N | M+B | B+N | M+N | M+B+N |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Experimental CMYo (NmL/g VSadded) |  | 274.43 ± 0.87 | 293.92 ± 1.12 | 304.66 ± 0.43 | 296.88 ± 0.68 | 306.61 ± 1.15 | 324.83 ± 3.78 | 302.58 ± 2.05 | 305.28 ± 0.33 |
| FM | Ym (NmL/g VSadded) | 281.80 | 301.75 | 308.45 | 295.93 | 314.29 | 307.31 | 326.82 | 310.83 |
|  | Dif (%) | 2.69 | 2.67 | 1.24 | -0.32 | 2.51 | 1.56 | 0.61 | 1.82 |
|  | k (day-1) | 0.112 | 0.127 | 0.116 | 0.121 | 0.121 | 0.122 | 0.116 | 0.118 |
|  | R2 | 0.9822 | 0.9841 | 0.9865 | 0.9872 | 0.9867 | 0.9890 | 0.9890 | 0.9910 |
|  | RMSE | 10.79 | 10.67 | 10.00 | 9.20 | 10.14 | 8.88 | 9.45 | 8.14 |
|  | AIC | 102.63 | 102.20 | 99.61 | 96.28 | 100.14 | 94.85 | 97.36 | 91.37 |
| MGM | Ym (NmL/g VSadded) | 265.08 | 285.12 | 290.03 | 278.45 | 297.01 | 290.34 | 307.11 | 293.42 |
|  | Dif (%) | -3.41 | -3.00 | -4.80 | -6.21 | -3.13 | -4.04 | -5.45 | -3.89 |
|  | Rm (NmL/g VSadded/day) | 23.56 | 26.83 | 24.62 | 24.68 | 26.06 | 25.56 | 26.18 | 24.93 |
|  | λ (day) | 0.44 | 0.19 | 0.00 | 0.00 | 0.04 | 0.00 | 0.00 | 0.00 |
|  | R2 | 0.9891 | 0.9885 | 0.9833 | 0.9762 | 0.9870 | 0.9818 | 0.9810 | 0.9842 |
|  | RMSE | 8.42 | 9.09 | 11.14 | 12.55 | 10.00 | 11.42 | 12.45 | 10.77 |
|  | AIC | 95.91 | 98.96 | 107.09 | 111.87 | 102.75 | 108.10 | 111.52 | 105.74 |

Note: CMYo = the actual observed cumulative methane yield (NmL CH4/g VSadded); Ym = Specific methane yield of substrates (NmL CH4/g VSadded); e = 2.718; k = Hydrolytic rate constant (day-1); t = Digestion time (days); Rm = Maximum methane production rate (NmL CH4/g VSadded/day); λ = Lag phase (days).

Fig. S3. The calculated increased value of the Co-AD process (M, microaeration; B, biochar addition; N, nanoparticles; and their combinations) in the enhanced treatments grouped by different technologies compared to the control (0 line) without enhancing technologies, variation with the proposed operational time (solid retention time, days)