Supporting Information of

Kinetics of air mixing–anaerobic digestion: High organic substrate concentrations enhance system resistance to oxygen stress

Kailun Songa,b, Huan Chena,b, Yue Zhaoa,b, Li Chena,b, Rong Youa,b, Yilin Wanga,b, Hongnan Yanga,b, Wenguo Wanga,b, Liangwei Denga,b*

aBiogas Institute of Ministry of Agriculture and Rural Affairs, Chengdu 610041, P. R. China

bLaboratory of Development and Application of Rural Renewable Energy, Ministry of Agriculture and Rural Affairs, Chengdu 610041, P. R. China

*Corresponding author:

E-mail: dengliangwei@caas.cn

Tel/Fax: +86-28-85236376

Number of pages: 9

Number of Texts: 1

Number of Tables: 5

Number of Figures: 1

Content

Texts:

Text S1. Experiment of methane production model independent validation

Tables:

Table S1. Characteristics of swine wastewater and anaerobic inoculum sludge

Table S2. Maximum values of ORP and DO content in air mixing-anaerobic digestion

Table S3. Rates of ORP increase (RORP) and decrease (UORP) in air mixing-anaerobic digestion

Table S4. Methane production rates in the abnormal, promotion and normal phases

Table S5. Predicted and measured methane outputs in air mixing-anaerobic digestion at various mixing time at an intensity of 66.7 mL/(L·min)

Figures:

Figure S1. Variations in biogas output at different organic substrate concentrations and different air mixing intensities.

Supporting Texts

Text S1. Experiment of methane production model independent validation

Independent validation of the methane production model was conducted using swine wastewater at a substrate concentration of 5 g COD/L. Three air-mixing intensities were applied (33.3, 66.7, and 100 mL air/min), along with a control group without air mixing. The mixing duration was set to 120 min, and each treatment was performed in triplicate. Biogas production over 24 h was subsequently collected, methane concentration was measured, and the actual methane yield was calculated accordingly.

Supporting Tables

Table S1. Characteristics of swine wastewater and anaerobic inoculum sludge

| Parameter | Swine wastewater | Sludge inoculum |
| --- | --- | --- |
| Total solid (TS) (g/L) | 1.73 ±0.41 | 88.53 ±0.53 |
| Volatile solid (VS) (g/L) | 0.43 ±0.18 | 51.64 ±1.01 |
| COD (g/L) | 9.97 ±0.32 |  |
| pH | 7.2 ±0.2 | 7.1 ±0.2 |
| ORP (mV) | -465.7 ±8.7 |  |

Table S2. Maximum values of ORP and DO content in air mixing-anaerobic digestion

|  | ORPmax (mV) |  |  | DO (mg/L) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| g COD/L | 33.3 mL air/min | 66.7 mL air/min | 100 mL air/min | 33.3 mL air/min | 66.7 mL air/min | 100 mL air/min |
| 2 | -244 ±2.12 | -219 ±3.87 | -120 ±1.44 | 0.181 ±0.005 | 0.245 ±0.013 | 0.800 ±0.016 |
| 4 | -275 ±2.95 | -235 ±1.24 | -136 ±1.37 | 0.125 ±0.006 | 0.203 ±0.010 | 0.662 ±0.031 |
| 6 | -310 ±4.60 | -249 ±3.43 | -195 ±2.69 | 0.082 ±0.004 | 0.171 ±0.008 | 0.328 ±0.015 |
| 8 | -327 ±2.19 | -276 ±3.85 | -237 ±3.47 | 0.067 ±0.003 | 0.123 ±0.006 | 0.198 ±0.009 |
| 10 | -357 ±3.76 | -296 ±4.24 | -259 ±3.37 | 0.046 ±0.002 | 0.097 ±0.005 | 0.151 ±0.007 |

Table S3. Rates of ORP increase (RORP) and decrease (UORP) in air mixing-anaerobic digestion

|  | RORP (mV/min) |  |  | UORP (mV/min) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| g COD/L | 33.3 mL air/min | 66.7 mL air/min | 100 mL air/min | 33.3 mL air/min | 66.7 mL air/min | 100 mL air/min |
| 2 | 6.72 ±0.169 | 11.0 ±0.247 | 17.1 ±0.274 | 0.595 ±0.032 | 0.688 ±0.125 | 0.993 ±0.027 |
| 4 | 6.08 ±0.192 | 9.79 ±0.226 | 15.9 ±0.249 | 0.714 ±0.155 | 0.963 ±0.069 | 1.15 ±0.033 |
| 6 | 5.83 ±0.212 | 9.59 ±0.247 | 14.8 ±0.304 |  | 1.07 ±0.114 | 1.30 ±0.067 |
| 8 | 6.07 ±0.248 | 9.45 ±0.280 | 11.0 ±0.269 |  | 1.24 ±0.281 | 1.32 ±0.114 |
| 10 | 5.82 ±0.308 | 9.07 ±0.301 | 10.9 ±0.295 |  | 1.43 ±0.19 | 1.46 ±0.149 |

Table S4. Methane production rates in the abnormal, promotion and normal phases

|  | Abnormal phase (mL/h) |  |  | Promotion phase (mL/h) |  |  | Control/ Normal phase (mL/h) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| g COD/L | 33.3 mL air/min | 66.7 mL air/min | 100 mL air/min | 33.3 mL air/min | 66.7 mL air/min | 100 mL air/min |  |
| 2 | 7.10 ±0.251 | 5.27 ±0.184 | 3.15 ±0.110 | 17.8 ±0.255 | 12.1 ±0.381 | / | 18.0 ±0.056 |
| 4 | 7.30 ±0.314 | 7.24 ±0.218 | 4.07 ±0.167 | 28.7 ±1.10 | 14.7 ±0.556 | / | 25.2 ±0.964 |
| 6 | 21.2 ±1.54 | 21.3 ±1.32 | 17.3 ±1.34 | 47.8 ±0.99 | 39.8 ±2.01 | 27.3 ±1.02 | 42.9 ±0.309 |
| 8 | 27.4 ±1.42 | 23.0 ±1.35 | 24.0 ±2.37 | 57.0 ±3.87 | 50.7 ±1.78 | 30.3 ±1.46 | 49.0 ±0.750 |
| 10 | 41.1 ±1.88 | 38.1 ±1.74 | 37.1 ±1.69 | 97.9 ±4.47 | 89.4 ±4.08 | 79.3 ±3.62 | 70.1 ±3.20 |

Table S5. Predicted and measured methane outputs in air mixing-anaerobic with various durations of air mixing at an intensity of 66.7 mL/(L·min)

| Air mixing duration (tm) (min) | Predicted methane production (mL/d) | Measured methane production (mL/d) | Relative error (%) |
| --- | --- | --- | --- |
| 1 | 5446 | 5447±117 | -0.0177 |
| 2 | 5439 | 5351±116 | 1.63 |
| 3 | 5419 | 4952±55 | 8.61 |
| 5 | 3937 | 4570±64 | -14.7 |
| 10 | 3904 | 4330±56 | -9.67 |
| 30 | 3835 | 4029±93 | -4.11 |
| 60 | 3739 | 3977±119 | -5.66 |
| 120 | 3599 | 3939±134 | -8.85 |

Supporting figures

Figure S1. Variations in biogas output at different organic substrate concentrations and different air mixing intensities (a, Control; b, 33.3 mL air/min; c, 66.7 mL air/min; d, 100 mL air/min).