Supplementary Information

Treatment of fresh leachate by microaeration pretreatment combined with IC-AO² process: performance and mechanistic insight

Mengqi Wei a, Ruina Zhang c,d, Mingjian Zhou a, Zhiqiang Yuan a, Haiping Yuan a, Nanwen Zhua,b*

a School of Environmental Science & Engineering, Shanghai Jiao Tong University, Shanghai, 200240, China

b Shanghai Engineering Research Center of Solid Waste Treatment and Resource Recovery, Shanghai, 200240, China

c Shanghai Institute for Design & Research on Environmental Engineering Co., Ltd, Shanghai, 200040, China

d Shanghai Environmental Sanitation Engineering Design Institute Co., Ltd, Shanghai, 200001, China

*Corresponding Author: E-mail: nwzhu@sjtu.edu.cn; Fax: +86 21 54742817.

Content:

2 texts

7 tables

1 figure

Text S1. Sample pretreatment and SPE method for quantification of antibiotics

Filtered samples（200ml）were acidified with hydrochloric acid to pH 3. 0.2 g Na2EDTA chelating agent and internal standard (50 ng for each sample) were added to samples. HLB cartridges (500 mg, 6 mL, Waters) were firstly activated with 10 mL methanol (MS grade), 10 mL ultra-pure water and 10 mL acid (pH=3); Leachate samples were add to cartridges at a flow rate of 2 mL/min, then rinse the cartridges with 20 mL HCl (pH=3) to remove the salts; After that, cartridges were dried with a stream of N2 and eluted with 1 mL of methanol and kept at 4 ℃ in dark until tested.

Text S2. Analysis of the antibiotics

For analysis of the antibiotics, 18 target compounds were quantified by the external standard method, and the correlation coefficients (r²) of calibration curves for all the target antibiotics were typically above 0.99. Mass spectrometry conditions: ESI positive/negative ion mode, curtain gas of 35 psi, ionization voltage was 5500V-4500V, desolventizing temperature was 500 ℃, atomizing gas of 55 psi, auxiliary heating gas of 55 psi, multiple reaction monitoring (MRM) was used to analyze the sample. Liquid chromatography conditions: methanol solvent, column was ACQUITY BEH C18 2.1*100 mm 1.7 um, temperature was 40 ℃, mobile phase A was 0.1% formic acid + pure water, mobile phase B was 0.1% formic acid + acetonitrile.

Table S1. The physicochemical properties of target antibiotics in this study.

| Compound | Abbreviation | Molecular formula | Molecular weight | Structure |
| --- | --- | --- | --- | --- |
| Sulfadiazine | SDZ | C10H10N4O2S | 250.28 |  |
| Sulfathiazole | STZ | C9H9N3O2S2 | 255.32 |  |
| Sulfapyridine | SPZ | C11H11N3O2S | 249.29 |  |
| Sulfamerazine | SMZ | C11H12N4O2S | 264.30 |  |
| Sulfamethazine | SMT | C12H14N4O2S | 278.34 |  |
| Sulfaquinoxaline | SQ | C14H12N4O2S | 300.34 |  |
| Oxytetracycline | OTC | C22H24N2O9 | 460.15 |  |
| Tetracycline | TC | C22H24N2O8 | 444.15 |  |
| Doxycycline | DOC | C22H24N2O8 | 444.43 |  |
| Chlortetracycline | CTC | C22H23ClN2O8 | 478.11 |  |
| Erythromycin | ETM | C37H67NO13 | 733.93 |  |
| Roxithromycin | RTM | C41H76N2O15 | 837.05 |  |
| Ofloxacin | OFX | C18H20FN3O4 | 329.34 |  |
| Norfloxacin | NFX | C16H18FN3O3 | 319.33 |  |
| Ciprofloxacin | CFX | C17H18FN3O3 | 331.34 |  |
| Enrofloxacin | EFX | C19H22FN3O3 | 359.40 |  |
| Chloramphenicol | CAP | C11H12Cl2N2O5 | 323.13 |  |
| Thiamphenicol | TAP | C12H15Cl2NO5S | 356.22 |  |

Table S2. Elution gradient of liquid chromatography.

| Time (min) | A(%) | B(%) | Flow rate (mL/min) |
| --- | --- | --- | --- |
| 0.0 | 90 | 10 | 0.4 |
| 0.5 | 90 | 10 | 0.4 |
| 3.0 | 50 | 50 | 0.4 |
| 6.0 | 0 | 100 | 0.4 |
| 8.0 | 0 | 100 | 0.4 |
| 8.1 | 90 | 10 | 0.4 |
| 10.0 | 90 | 10 | 0.4 |

A and B was mobile phase

A: 0.1% formic acid + pure water

B: 0.1% formic acid + acetonitrile

Table S3. Tests of Between-Subjects Effects (COD)

| Dependent Variable: COD |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Source | Type III Sum of Squares | df | Mean Square | F | Sig. |
| Corrected Model | 4000036.833a | 8 | 500004.604 | 149.438 | .000 |
| Intercept | 76842756.000 | 1 | 76842756.000 | 22966.302 | .000 |
| R | 1196126.042 | 2 | 598063.021 | 178.745 | .000 |
| RD | 69666.833 | 2 | 34833.417 | 10.411 | .000 |
| HRT | 616295.250 | 2 | 308147.625 | 92.097 | .000 |
| SRT | 1544270.458 | 2 | 772135.229 | 230.771 | .000 |
| R * RD | .000 | 0 | . | . | . |
| R * HRT | .000 | 0 | . | . | . |
| R * SRT | .000 | 0 | . | . | . |
| RD * HRT | .000 | 0 | . | . | . |
| RD * SRT | .000 | 0 | . | . | . |
| HRT * SRT | .000 | 0 | . | . | . |
| R * RD * HRT | .000 | 0 | . | . | . |
| R * RD * SRT | .000 | 0 | . | . | . |
| R * HRT * SRT | .000 | 0 | . | . | . |
| RD * HRT * SRT | .000 | 0 | . | . | . |
| R * RD * HRT * SRT | .000 | 0 | . | . | . |
| Error | 210791.167 | 63 | 3345.892 |  |  |
| Total | 130743060.000 | 72 |  |  |  |
| Corrected Total | 4210828.000 | 71 |  |  |  |

a. R Squared = .950 (Adjusted R Squared = .944)

Table S4. Tests of Between-Subjects Effects (TN)

Dependent Variable: TN

| Source | Type III Sum of Squares | df | Mean Square | F | Sig. |
| --- | --- | --- | --- | --- | --- |
| Corrected Model | 326881.270a | 8 | 40860.159 | 143.419 | .000 |
| Intercept | 4225217.284 | 1 | 4225217.284 | 14830.535 | .000 |
| R | 71399.763 | 2 | 35699.881 | 125.307 | .000 |
| RD | 41153.490 | 2 | 20576.745 | 72.224 | .000 |
| HRT | 10359.317 | 2 | 5179.658 | 18.181 | .000 |
| SRT | 55900.893 | 2 | 27950.446 | 98.106 | .000 |
| R * RD | .000 | 0 | . | . | . |
| R * HRT | .000 | 0 | . | . | . |
| R * SRT | .000 | 0 | . | . | . |
| RD * HRT | .000 | 0 | . | . | . |
| RD * SRT | .000 | 0 | . | . | . |
| HRT * SRT | .000 | 0 | . | . | . |
| R * RD * HRT | .000 | 0 | . | . | . |
| R * RD * SRT | .000 | 0 | . | . | . |
| R * HRT * SRT | .000 | 0 | . | . | . |
| RD * HRT * SRT | .000 | 0 | . | . | . |
| R * RD * HRT * SRT | .000 | 0 | . | . | . |
| Error | 17948.691 | 63 | 284.900 |  |  |
| Total | 7030929.176 | 72 |  |  |  |
| Corrected Total | 344829.961 | 71 |  |  |  |

a. R Squared = .948 (Adjusted R Squared = .941)

Table S5. Tests of Between-Subjects Effects (NH4+-N)

Dependent Variable: Ammonia

| Source | Type III Sum of Squares | df | Mean Square | F | Sig. |
| --- | --- | --- | --- | --- | --- |
| Corrected Model | 201236.256a | 8 | 25154.532 | 61.098 | .000 |
| Intercept | 3246176.941 | 1 | 3246176.941 | 7884.600 | .000 |
| R | 46796.358 | 2 | 23398.179 | 56.832 | .000 |
| RD | 19982.066 | 2 | 9991.033 | 24.267 | .000 |
| HRT | 2910.809 | 2 | 1455.405 | 3.535 | .035 |
| SRT | 2929.775 | 2 | 1464.888 | 3.558 | .034 |
| R * RD | .000 | 0 | . | . | . |
| R * HRT | .000 | 0 | . | . | . |
| R * SRT | .000 | 0 | . | . | . |
| RD * HRT | .000 | 0 | . | . | . |
| RD * SRT | .000 | 0 | . | . | . |
| HRT * SRT | .000 | 0 | . | . | . |
| R * RD * HRT | .000 | 0 | . | . | . |
| R * RD * SRT | .000 | 0 | . | . | . |
| R * HRT * SRT | .000 | 0 | . | . | . |
| RD * HRT * SRT | .000 | 0 | . | . | . |
| R * RD * HRT * SRT | .000 | 0 | . | . | . |
| Error | 25937.795 | 63 | 411.711 |  |  |
| Total | 5819055.460 | 72 |  |  |  |
| Corrected Total | 227174.051 | 71 |  |  |  |

a. R Squared = .886 (Adjusted R Squared = .871)

Table S6. Tests of Between-Subjects Effects (NO3--N)

Dependent Variable: Nitrate

| Source | Type III Sum of Squares | df | Mean Square | F | Sig. |
| --- | --- | --- | --- | --- | --- |
| Corrected Model | 2398.471a | 8 | 299.809 | 6.848 | .000 |
| Intercept | 30245.848 | 1 | 30245.848 | 690.847 | .000 |
| R | 132.421 | 2 | 66.210 | 1.512 | .228 |
| RD | 942.803 | 2 | 471.401 | 10.767 | .000 |
| HRT | .489 | 2 | .244 | .006 | .994 |
| SRT | 88.733 | 2 | 44.367 | 1.013 | .369 |
| R * RD | .000 | 0 | . | . | . |
| R * HRT | .000 | 0 | . | . | . |
| R * SRT | .000 | 0 | . | . | . |
| RD * HRT | .000 | 0 | . | . | . |
| RD * SRT | .000 | 0 | . | . | . |
| HRT * SRT | .000 | 0 | . | . | . |
| R * RD * HRT | .000 | 0 | . | . | . |
| R * RD * SRT | .000 | 0 | . | . | . |
| R * HRT * SRT | .000 | 0 | . | . | . |
| RD * HRT * SRT | .000 | 0 | . | . | . |
| R * RD * HRT * SRT | .000 | 0 | . | . | . |
| Error | 2758.190 | 63 | 43.781 |  |  |
| Total | 59109.501 | 72 |  |  |  |
| Corrected Total | 5156.661 | 71 |  |  |  |

a. R Squared = .465 (Adjusted R Squared = .397)

Table S7. Molecular parameters of leachate samples derived from assigned molecular formulae (Raw: raw leachate; MA: MA effluent; IC: IC effluent; Effluent: AO² effluent).

| Sample | Cavg | Havg | Oavg | Navg | Savg | DBEavg | H/Cavg | O/Cavg | m/zavg |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Raw | 19.9 | 30.8 | 5.5 | 0.7 | 0.7 | 6.1 | 1.5 | 0.3 | 399.7 |
| MA | 17.5 | 24.2 | 6.7 | 0.7 | 0.7 | 7.0 | 1.4 | 0.4 | 383.5 |
| IC | 16.7 | 23.2 | 7.2 | 0.7 | 0.7 | 6.7 | 1.4 | 0.4 | 378.8 |
| Effluent | 17.8 | 23.5 | 7.6 | 0.9 | 0.6 | 7.7 | 1.3 | 0.5 | 396.2 |

Fig. S1. Broad-band mass spectra of the C18 extracted leachate samples using negative-ion ESI FT-ICR MS.